Draft Stormwater Geodata Transfer Standard

As developed by the Metro Stormwater Geodata Project

Released for Public Stakeholder Review: April 17, 2020

This draft of the Stormwater Geodata Transfer Standard, v. 0.5

is published for public statewide stakeholder review and comment.

Funding for this project was provided by the

Water Resources Center of the University of Minnesota

https://www.wrc.umn.edu/

This document was compiled and edited by Geoffrey Maas from Ramsey County Information Services Department and Carrie Magnuson of the Ramsey Washington Metro Watershed District on behalf of the membership of the Metro Stormwater Geodata Project (MSWGP) Steering Committee.

Stormwater Geodata Transfer Standard: Draft Version 0.5 – For Public Review and Comment

Table of Contents
Note: Each entry in Table of Contents is linked to its corresponding page in the document, simply Control-click the entry to go to the page.

Overview and Context3

Project Contacts6

Stormwater Standard – Components 6

Stormwater Standard – Elements11

Pilot Study Area – Sample Dataset Available 12

Stormwater Standards – Guide to Schema Attributes 13

“Inclusion” Category Explanations 14

Pipe Components 15

Channel Components 27

Artificial Path Components 38

Basin Components 43

Hydraulic Control Structures Components 56

Pollution Control Structures Components 68

Artificial Point Components 79

Inlet Components 84

Outlet Components  97

Manhole Components 111

Lift Station Components 123

Best Management Practices Components 136

Monitoring Components 149

Basins (As Polygon Features) 159

Best Management Practices (BMPs as Polygon Features) 160

Treatment of Polygons in the Draft Standard 161

Acknowledgements

Huge thanks are due to the numerous participants of the Metro Stormwater Geodata Project (April 2018 – present) for their time, energy, patience and thoughtful contributions to the development of this work. Enormous thanks are due to John Bilotta and Jeff Peterson of the Water Resources Center at the University of Minnesota for their facilitation of grant funding for our pilot dataset, to Tami Maddio (City of Eagan), Bryan Pittman and Kyle Seifert (WSB Engineering), Erik Madland and Brian Gruidl (City of Bloomington) and Alex Blenkush (Hennepin County) for their guidance, enthusiasm and ‘can-do’ attitude in bringing the sample pilot dataset to fruition. Big thanks also to Masha Hoy (Carver County Water Resources), Delaney Moberly (SRF Consulting Group), and David Malm (Bolton and Menk Engineering) for their generous contribution of photographs of stormwater assets for this document. Additional thanks to the following people whose past and current contributions and support are essential to this project taking shape: Doug Prazak (Dakota County Land Records), Jon Hoekenga (Metropolitan Council), Ann Houghton (Hennepin County [retired]) and to the members of the MetroGIS Coordinating Committee for their continued support of this effort as a regional priority project.

Overview, Context and FAQs

Purpose of this document.

The purpose of this document is two-fold; first, it is intended to provide an information-rich resource document about the content of the draft Stormwater Geodata Transfer Standard as developed by the participants of the Metro Stormwater Geodata Project (April 2018 – present) and secondly, to serve as a resource for the entire geospatial, stormwater, engineering, water resources and planning community to reference and review so they can be better prepared to provide comments, suggestions and recommendations for the improvement of the draft standard in development.

What is the Stormwater Geodata Transfer Standard (SGTS)?

This SGTS is simply a set of attribute specifications such as field name, field type, field length and field order as well as a set of standardized terminology and domain values to serve as a means for the translation and aggregation of stormwater geospatial data into a common format.

What is the Metro Stormwater Geodata Project (MSWGP)?

This MSWGP is a voluntary collaborative project comprised of private sector and public sector partners in the Twin Cities metropolitan region. The goal of the MSWGP effort is to create a stormwater geodata transfer standard that reflects the functional needs of the professional community and that contains the attributes, terminology and content—as well as the supporting documentation—to satisfactorily meet those needs.

The MSWGP began with a kick-off meeting in Medina, Minnesota on April 17, 2018. The primary purpose of this kick-off session was to:

- to engage a variety of interested stakeholders

- to present the proposed project and articulate some of the needs for it

- to document the core business needs of the participants

- to determine if there was enough interest from the stakeholder community in the project; and,

- if so, to create an initial Steering Committee to begin developing it.

The MSWGP builds upon initial prior work undertaken between 2008-2010 by state level agencies to create an initial stormwater system data exchange standard. The current MSWGP effort is being co-coordinated by volunteer staff from metro counties and watershed districts who organize and lead the meetings, prepare needed research and contextual materials and document the input from the participants.

The MSWGP Steering Committee—comprised of professionals with backgrounds in engineering, planning, landscape architecture, water resources management, water quality regulation, monitoring, geospatial work, asset management, public works and other disciplines—convened six times between June 2018 and October 2019. In addition to the project Steering Committee, several smaller technical working groups also met to confer on specific details of the standard’s development; the result of these groups’ dedicated discussion, focus and work is contained within the pages of this document.

Version 0.5 of the Draft Stormwater Geodata Transfer Standard

This document represents the current iteration of the standard in its development. This is draft—called ‘version 0.5’ (or affectionately just the ‘dot five’)—represents the results of eighteen months of work, discussion, research, revision and review during 2018 and 2019 by the MSWGP participants. By early 2020, the MSWGP Steering Committee felt that a good enough draft was prepared which could be put before the entire statewide stakeholder professional community to solicit their review, comments and suggestions for improvement.

What need or purpose does this proposed standard fulfill?

The Twin Cities metropolitan region is comprised of 186 municipal governments, 7 county governments, 33 watershed management units, numerous state and regional interests, educational campuses and private interests all of whom build, maintain, own and manage surface and subsurface stormwater fixtures and conveyance infrastructure.

As no formally approved or agreed-upon geospatial data standard for creating and maintaining the digital representations of these features exists, each of these agencies has come up with its own unique schema for representing, attributing and maintaining their stormwater assets.

Over the years, this has resulted in as many different schema types as there are agencies maintaining them. This lack of data standardization has led to significant challenges when attempts to combine and align data from several jurisdictions for mapping, modeling and analysis are undertaken. The purpose of this standard is to provide a single, commonly accepted set of attribute specifications (field name, field type, field width, field order, domain values) for transferring data between interests and aggregating stormwater geospatial data from many sources into a common format. The eventual adoption and usage of this standard is intended to facilitate and enhance the ability of geospatial practitioners to share data and to reduce incompatibilities when acquiring, processing, aggregating and disseminating stormwater system geodata.

Additional intended goals for developing a stormwater geodata transfer standard include better integration with asset management software applications, facilitation of flow modeling for water management and emergency response uses, ability to determine ownership and maintenance responsibility of stormwater assets, enhance potential analysis capacity for determining efficacy of installed best management practices, provide the ability to link geodata features to inspection and regulatory reports, ability to edge match features along boundaries and many others.

Is the Stormwater Geodata Transfer Standard a mandated standard? Is this something we are going to be required to use? Absolutely not. No data standard developed by and for the geospatial of community in Minnesota are required or mandated. Standards—such as this emerging stormwater geodata transfer standard—are intended to serve as voluntary tools that the members of our profession work to develop collaboratively as a means of working more easily and efficiently with one another. There are no laws, statues, administrative rules or court orders in Minnesota that dictate what a city, county, watershed district or other agency must do with their data in terms of using or maintaining standards. The use of this forthcoming standard and other geodata standards, such as those already adopted for address points, road centerlines and parcels—while encouraged—are completely voluntary.

Is this version 0.5 the “definitive version” of the Stormwater Geodata Transfer Standard?

No. This draft standard has been prepared by the participants of the Metro Stormwater Geodata Project as a beginning point—this initial ‘version 0.5’ is being submitted and offered to the entire stakeholder community in Minnesota for them to review, critique, provide suggestions and comments on it. This review period is anticipated to take place during the spring and summer of 2020, and the comments received will be documented, reviewed and considered for modifying the next iteration of the standard. Data standards are strengthened by both the review and input of the professional community and through their usage.

My agency already has geospatial data representing our stormwater network that we use to meet our needs. Why should we care about this newly developing standard?

An agency or interest already creating and managing its data in its own format or schema can certainly maintain its data in its own format. The MSWGP effort is simply proposing and advancing this standard as a transfer standard to meet a variety of needs as expressed by the wider user community. It is our hope that agencies and interests creating or consuming stormwater geodata would consider at very least providing comment on how the proposed v. 0.5 standard might be revised or edited to better meet their existing needs and use cases.

Why go through all the trouble of creating a new and different stormwater geodata standard?

Why doesn’t everyone simply just use the ESRI Stormwater Data Model?

The v. 0.5 draft Stormwater Geodata Transfer Standard offered by the MSWGP was in many ways inspired by the ESRI Stormwater Geodata Model, as the v. 0.5 uses many of the same terms, concepts, domain values and data categories and has many other similarities to the ESRI model. The key differences are that the v. 0.5 was created to address many additional specific use cases and needs specifically expressed by the user community in Minnesota, such as integration with asset management software, MS4 compliance reporting, integration with field collection methods, flow modeling needs and others. Comparison with, and translation between the ESRI model and the v. 0.5 is anticipated as one of the tests to be conducted during 2020 stakeholder review period. Any agency already using the ESRI Stormwater Model will see the clear similarities between their existing data and the v. 0.5 draft schema format.

Applicability of this standard.

Agencies who produce and maintain geospatial data representing stormwater features are certain to have unique methods, definitions, and criteria for capture and storage of geospatial data representing stormwater features to satisfy their own business requirements and meet their internal agency needs.

This standard simply seeks to establish a set of attribute specifications primarily intended for data exchange purposes. This proposed and emerging standard may be used not only to transfer and aggregate data, but also potentially utilized to create, manage and maintain geospatial data for representing stormwater fixtures, assets and conveyance systems within a jurisdiction. This standard in no way attempts to define, change or dictate any agency’s existing internal data capture or storage specifications; however, some data producers may find benefit in using the standard to manage and maintain their data.

What does the public stakeholder review of the v. 0.5 draft Stormwater Geodata Standard entail?

The MSWGP team will be collecting and documenting the comments, suggestions, revisions and recommendations received during calendar 2020 on the draft standard. The MSWGP team will use these comments to revise, improve and edit the version 0.5 standard, hopefully improving it and refining it to better satisfy the various needs of the stakeholder community. The creation of geodata standards, including this one for stormwater in Minnesota is best accomplished by using an inclusive and transparent process that encourages input and participation by the entire professional community. The resulting standard which results is a resource that reflects the expertise, needs and intelligence of the professionals who need it and (hopefully) will make use of it.

How are the various stormwater fixtures represented in this draft standard?

There are thirteen (13) unique categories of representation, three as linear features, the remaining ten as point features. Additionally, there is capacity to illustrate both Basins and Best Management Practices (BMPs) as polygon features within the draft standard, this is explained in more detail in the ‘Stormwater Standard – Components’ section beginning on Page 7 of this document. The table below outlines the general intention of how the data is organized by geometry type in the draft standard.

|  | Representational Type |  |  |
| --- | --- | --- | --- |
| Stormwater Feature Type | Point | Line (Polyline) | Polygon |
| Pipes |  | Primary |  |
| Channels |  | Primary |  |
| Artificial Paths |  | Primary |  |
| Basins | Primary |  | Secondary |
| Hydraulic Control Structures | Primary |  |  |
| Pollution Control Structures | Primary |  |  |
| Artificial Points | Primary |  |  |
| Inlets | Primary |  |  |
| Outlets | Primary |  |  |
| Manholes | Primary |  |  |
| Lift Stations | Primary |  |  |
| Best Management Practices (BMPs) | Primary |  | Secondary |
| Monitoring | Primary |  |  |

Project Contacts

Who do I contact if I have questions about this standard or I wish to provide comments on it?

Please contact the following individuals—who are serving as co-coordinators of the Metro Stormwater Geodata Project—they will field your questions and will gladly add your comments to those documented during the v. 0.5 stakeholder review period during 2020.

Geoffrey Maas, Senior Geospatial Business Analyst	Carrie Magnuson, GIS Technician

Ramsey County					Ramsey-Washington Metro Watershed District

geoffrey.maas@co.ramsey.mn.us			carrie.magnuson@rwmwd.org

Alex Blenkush, GIS Analyst

Hennepin County

alex.blenkush@hennepin.us

Stormwater Standard – Components

Stormwater fixtures vary widely in both their type and function. This variation provides a tremendous challenge to the task of capturing and correctly representing these features with geospatial data. The draft Stormwater Geodata Transfer Standard makes use of the following fifteen data types (referred to as Components) for representing stormwater features and assets. Each has been assigned a color, with which it will be associated consistently throughout this and other accompanying documentation.

Components appearing in the draft Stormwater Geodata Transfer Standard:

Pipes (Linear features)

Channels (Linear features)

Artificial Paths (Linear features)

Basins (Point features)

Hydraulic Control Structures (Point features)

Pollution Control Structures (Point features)
Artificial Points (Point features)

Inlets (Point features)

Outlets (Point features)

Manholes (Point features)

Lift Stations (Point features)

Best Management Practices (BMPs) (Point features)

Monitoring Sites (Point features)

Basins (represented as polygons)

Best Management Practices (represented as polygons)

Each of the Components are briefly described below. In reference to their applicability to the draft Stormwater Geodata Transfer Standard, the most broad and inclusive intent of the definition is intentional.

Pipes

A pipe is a tube comprised of concrete, metal, plastic or other material that is installed to convey stormwater; pipes are represented with line geometry (polylines) and may vary in their dimension, type, shape, compositional material and lining material.

Channels

A channel is a constructed waterway or natural waterway (either in its original state or modified by human agency) with the primary design purpose to convey stormwater runoff. Channels are represented with line geometry (polylines) and may vary in their dimension, type, shape and lining material.

Artificial Paths

An artificial path is a linear feature intended to represent a line of flow through a water body or overland where no such actual physical pipe, channel or path exists.  Artificial paths are represented with line geometry (polylines) and may vary in their type and intended usage and purpose.

Basins

A basin is defined as a site, fixture or natural feature with the capacity to act as a temporary or permanent pool of water as its primary function or intended design. Basins may vary in their type, origin, function and impairment category. Basins are represented with point geometry.

(see also Basins (represented as polygons))

Hydraulic Control Structures

A hydraulic control structure is defined as a fixture installed with the intention to divert, disrupt or halt the flow of stormwater. Hydraulic control structures are represented with point geometry and can vary by type (e.g. dam, deck drain, diversion chamber, weir, etc.).

Pollution Control Structures

A pollution control structure is defined as a fixture installed with the specific intention to reduce or eliminate a water pollutant. Pollution control structures are represented with point geometry and can vary by type (e.g. grit chamber, filtration device, settling device, etc.).

Artificial Point

An artificial point is a point feature intended to represent phenomena that are useful for modeling but do not represent an actual physical asset. Examples include junction points, discharge points and centroids. Artificial points are represented with point geometry and can vary by type.

Inlet

An inlet is a site where stormwater enters the conveyance system. Inlets are represented with point geometry and can vary by both type and shape.

Outlet

An outlet is a site where stormwater discharges out from a pipe, channel or other conveyance. Outlets are represented with point geometry and can vary by both type and shape. Note, outlets are not to be confused with outfalls. An outfall is a type of outlet, specifically applied to where water discharges into the Waters of the United States (40 CFR 230.3(s)) or into another municipal separate storm sewer system (MS4) permittees jurisdiction.

Manhole

A manhole is an opening or access point to stormwater utilities which enables repair, maintenance, or inspection activities to take place. Manholes are represented with point geometry and can vary by type.

Lift Stations

A lift station is an installed structure to move stormwater from a lower elevation to a higher elevation where the site elevation or other situational context is not suited for gravity flow alone to move the water. Lift stations include several sub-components including receiving wells (wet wells), screens or filters for removing debris, pumps, valves, piping, control devices all of which may occur within a single enclosed structure. Lift stations are represented with point geometry and can vary by type.

Best Management Practices (BMPs)

Stormwater best management practices (BMPs) encompass a range of fixtures, devices, installations and other measures used to impound, detain, divert, hold, infiltrate and treat precipitation, snowmelt, surface runoff and other stormwater drainage with the intention of reducing the impact of pollutants contained in the water. Best management practices in their general type, filtration material and ground cover type. Basins are represented with point geometry (see also Best Management Practices (represented as polygons).

Monitoring Sites

Monitoring sites are locations where water monitoring activities are currently taking place or have taken place in the past at may include installation of a permanent device, temporary device or represent a single-event. Monitoring sites are represented by point geometry and may vary by category, type and status.

Basins (represented as polygons)

As listed above, a basin is defined as a site, fixture or natural feature with the capacity to act as a temporary or permanent pool of water as its primary function or intended design. Basins may vary in their type, origin, function and impairment category. In the draft Stormwater Geodata Transfer Standard, basins are primarily represented with point geometry, however, it is acknowledged that the representation of basins over and above a certain size as a polygon feature is necessary or a variety of visualization and analysis uses.

The area and extent of the basin feature may vary due to factors in precipitation, erosion or human agency in modifying the structural elements of the feature. In the example at right, the tan lines represent artificial paths, while the purple lines represent channels coming in and out of the basin and the green point (centroid) would remain the constant feature representing the basin itself—serving to maintain connectivity to the linear features as the basin may expand or contract over time. In (A) the shoreline (in light blue) has contracted, while in (B) the shoreline is larger.

As needed, new shorelines can be created (via heads-up digitizing, extraction from LIDAR, imported from CAD, GPS point collection on the perimeter of the basin, etc. – of note: a domain of ‘polygon creation method’ (PolyMethod) is provided in the draft Standard) but the linear and point features and their topological connection would remain intact to preserve the functionality of the geometry for flow modeling purposes.

Best Management Practices (represented as polygons)

Similar in concept to what was described above for basin data, BMPs would be carried as points; however, there is also capacity in the draft Stormwater Geodata Transfer Standard for creation and maintenance of BMPs as polygon features using the same method listed above.

Stormwater Standard – Elements

Each Component is further organized of up to seven different Elements. These Elements are simply a way of intuitively grouping the various individual attributes within each Component. The seven elements in use in the draft Stormwater Geodata Transfer Standard are as follows:

Identification Elements

Type Elements

Dimensional and Positional Elements

Origin and Status Elements

Capacity Elements

Data Elements

General Elements

Each of the Elements are briefly described and defined below. As with the Components, the most broad and inclusive intent of the definition of each Element is intentional. Elements are used to group the attributes in the Excel spreadsheet version of the standard and do not impact the content or functionality of their attributes.

Identification Elements

Identification elements contain the unique IDs, composite IDs and features the form or encompass things to be used as primary or foreign keys.

Type Elements

These elements contain information on the type, either specific or general, about the feature.

Dimensional and Positional Elements

These elements contain information on the dimensional attributes, materials, measurements and related information about the feature.

Origin and Status Elements

This element contains data on the origin, status, ownership of the features and links to other relevant documents.

Capacity Elements

These elements contain data on the capacity (e.g. volume) characteristics of the feature.

Data Elements

These elements contain information on the data representing the fixture, not the fixture itself. In the draft Stormwater Geodata Transfer Standard, any place the characters *_DA* appear in the database name, this is a hint that this attribute contains info about the data itself, not the physical stormwater fixture it represents.

General Elements

These elements indicate in what jurisdiction the fixture is located, codes for those jurisdictions and space for additional comments.

Pilot Study Area – Sample Dataset Available

To enhance the ability of the stakeholder community in becoming familiar with, to review and to test the

v. 0.5 draft Stormwater Geodata Transfer Standard, a sample dataset in the v. 0.5 schema format has been prepared and made available for download.

The sample pilot dataset contains two ‘storm-shed’ areas which drain to the Minnesota River, one in the City of Bloomington, the other in the City of Eagan. The sample pilot site includes a variety of the kinds of stormwater features the draft standard attempts to represent with geospatial data. Additionally, the Minnesota Department of Transportation also maintains several stormwater fixtures and assets in the pilot study area. The pilot site is shown in Exhibit A below.

Exhibit A - Pilot Study Area:

The sample pilot dataset encompasses a total area of 5.87 square miles (3,762 acres) encompassing portions of both Bloomington and Eagan. The dataset includes 49,279 linear feet (9.3 miles) of natural and artificial channels, 154,919 linear feet (29.3 miles) of stormwater pipes, 1122 acres of basins and open water, 58 outlet points, 1042 inlet points, 348 manholes/access points and 16 best practice installation sites.

Also, as no stormwater lift stations appear within designated sample pilot study site area, the City of Eagan has also included seven (7) lift station sites to the pilot dataset (outlined in the blue box). These lift station point sites are available with the downloadable sample pilot dataset.

The sample pilot dataset in the v. 0.5 draft standard format for the area above is available for download in both ESRI geodatabase format (.gdb) and shapefile format (.shp) from the following website:

https://www.metrogis.org/projects/stormsewers.aspx

Stormwater Standard – Guide to Schema Attributes

Each Component contains numerous individual attributes; this document will be organized so each attribute has its own descriptive entry which details its content and example of which is as follows with descriptions of each sub-component:

Element ID:	Example: L_PIPE1 - this is just a reference (marker) ID telling the user that the attribute is a linear feature (L), in the Pipe Component (PIPE) and is the first attribute in the list (1); the Element IDs will change with each version of the standard as attributes get added or dropped.

Attribute (Alias) Name:		Pipe ID

Database Name:		PIPE_ORID (these are kept to 10 characters or less)

Inclusion Category:		Mandatory, Conditional, If Available, Optional

These are the categories in use by standards adopted by the

Minnesota Geospatial Advisory Council

(Please see page 14 for explanation)

Field Width:			This indicates the character width of the field (a.k.a. Length)

Domain:			This indicates if the field has a domain of accepted values

Example:			Where applicable and example will be provided

Description:			An additional verbal description of fixture represented

Photographic or illustrative examples of the fixture or object being represented in the data schema may also be provided to offer context to the reader. The graphic below explains the table provided for each descriptor of each attribute presented in the draft v. 0.5 standard:

Element ID                        Name of attribute 	   	Inclusion Category information               Domain

Database name

`

Data type (Text, Float, Date, etc.)

Width

Examples (examples of actual values which might appear in the field)

Description

(descriptive characteristics, links to resources provided or other files are referenced as applicable)

For some attributes in this v. 0.5. draft standard, additional explanations, illustrations, graphics or photos may be provided to offer additional clarity.

“Inclusion” Categories Explained

“Inclusion” is a term used to explain the requirement for the population of a field in a dataset for it to comply with the standard.

Each attribute can be defined as of one of four types of Inclusion, these are:

- Mandatory;

- Conditional;

- If Available;

- Optional;

Each category is explained in more detail below:

Mandatory: In simple terms, field identified as ‘mandatory’ needs each record populated to be compliant with the standard.  The use of the term ‘mandatory’ indicates nothing more than for the data to be compliant with the standard it must have, at minimum, the mandatory fields populated. In a ‘mandatory’ designation, null values would not be allowed. Further, the term ‘mandatory’ is not to be applied to the fulfillment of an agency’s request for data (e.g. “it is ‘mandatory’ that you provide this data to us” – this is not the intention of the ‘mandatory’ category in this or any other standard).

Agencies can create and maintain data that does not contain the ‘mandatory’ fields in any standards. This simply means their data is not compliant with the standard, but in no way does it mean that their data isn’t useful or cannot be used.

For example, the field Pipe ID is a ‘mandatory’ field to be populated in the draft MSWGP v. 0.5 data standard, which makes sense as each asset should have its own unique identifier; however, a data producer such as a city may have digitized their stormwater network and while it has usable data, it simply has not applied a unique ID to each pipe in their system. Their data would not be compliant with the standard; but could still be used to fulfill many useful and important mapping and analysis uses.

Conditional: Each field with a ‘Conditional’ designation is to be populated with a non-null value for each record that is applicable to the feature or for which a specified condition exists. For example, the field Pipe Shape is a Conditional field; if the shape of a given pipe asset is not known, the data producer is can reasonably be expected to enter the value ‘Unknown’ and not to leave the field ‘blank’ or ‘null’.

If Available: Each field with an ‘If Available’ designation is to be populated—if the data exists—in the data provider’s database or system. If a data provider does not have the data, it cannot be populated. For example, the field Pipe Casing is an If Available field. As not all pipes types have or require a casing, or the presence of a casing may not be known, it is acceptable to leave unknowns as null values.

Optional: An ‘optional’ field is one that is not required to be populated, however, inclusion of this data would enhance the value and usability of the data and data producers are encouraged to provide as much data as possible.

The following sections break down the specific details of each attribute field. They are color-coded by type (Pipes in Blue, Channels in Violet, Basins in Green, etc.) in the document and the accompanying Excel spreadsheet documents.

Pipe Components

L_PIPE1 – Pipe ID

| Database Name | PIPE_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | STR7209708112209 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer |  |  |

L_PIPE2 – Pipe Federated ID

| Database Name | PIPE_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-STR7209708112209 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-STR7209708112209

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

STR7209708112209 = Example of the locally designated ID for the feature

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

L_PIPE3 – Pipe Type

| Database Name | PIPE_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 90 | Domain | PipeType |
| Examples | Pipe, perforated; Pipe, non-perforated; Underdrain, wrapped Underdrain, unwrapped; Drain tile, perforated; Drain tile, non-perforated; Culvert; Other; Unknown; Combined; Force Main-Pressurized; Cattle pass |  |  |
| Description | Generalized pipe type description |  |  |

L_PIPE4 – Pipe Shape

| Database Name | PIPE_SHP |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 24 | Domain | PipeShape |
| Examples | Arched, Barrel, Box, Circular, Cathedral, Cattle pass, Egg shaped, Elliptical, Horseshoe, Oblong, Rectangular, Round, Trapezoidal, Triangular, Tunnel, U-Shaped, Other, Unknown |  |  |
| Description | Predominant cross-sectional shape configuration of the pipe |  |  |

L_PIPE5 – Pipe Material

| Database Name | PIPE_MAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 30 | Domain | PipeMaterial |
| Examples | ABS Plastic, Asbestos Cement, Asphalt, Bituminous Fiber-Orangeburg, Brick, Brick Masonry, Cast Iron, Clay Tile (for complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |
| Description | Substance comprising a closed pipe |  |  |

L_PIPE6 – Pipe Lining

| Database Name | PIPE_LINE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 45 | Domain | PipeLining |
| Examples | Cured in Place, Fold and form, Deform/Reform, Segmented Panel, Segmented Pipe, Slip Lining, Spray Liner, Spiral Wound, Other, None, Unknown |  |  |
| Description | Pipe lining method |  |  |

L_PIPE7 – Pipe Diameter

| Database Name | PIPE_DIA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (numerical value in inches) |  |  |
| Description | Interior diameter of the pipe in inches Use decimal values; not fractions (e.g. value is 16.5 not 16 ½ or 16-½) Leave off name of measurement (e.g. value is 16.5, not 16.5” or 16.5 in.) |  |  |

L_PIPE8 – Pipe Equivalent

| Database Name | PIPE_EQA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | (numerical value in inches) |  |  |
| Description | Value equivalent to a measure of interior diameter—for a pipe that is not round or symmetrical—or if the pipe varies in width/shape from top to bottom - in inches |  |  |

L_PIPE9 – Pipe Height

| Database Name | PIPE_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | (numerical value in inches) |  |  |
| Description | Height of the interior of the pipe through its center in inches |  |  |

L_PIPE10 – Pipe Width

| Database Name | PIPE_WD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | (numerical value in inches) |  |  |
| Description | Width of the entire diameter (including pipe exterior) of the pipe in inches |  |  |

L_PIPE11 – Pipe Length

| Database Name | PIPE_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | (numerical value in feet) |  |  |
| Description | Length of the pipe in feet |  |  |

L_PIPE12 – Pipe Depth

| Database Name | PIPE_DEP |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | (numerical value in feet) |  |  |
| Description | Approximate depth of pipe in feet from the surface of the ground to the top of the pipe. Note: Depth of pipe from the surface or roadway may influence who conducts the maintenance on the pipe |  |  |

L_PIPE13 – Pipe Coverage

| Database Name | PIPE_CVG |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (numerical value in feet) |  |  |
| Description | A description of the type of ground cover above the pipe (this is useful for scoping and planning excavation and repair work) |  |  |

L_PIPE14 – Pipe Upstream Invert Elevation

| Database Name | PIPE_IELVU |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (numerical value in feet above mean sea level) |  |  |
| Description | Elevation at the bottom of the inside portion of the pipe, at the upstream point |  |  |

L_PIPE15 – Pipe Downstream Invert Elevation

| Database Name | PIPE_IELVD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (numerical value in feet above mean sea level) |  |  |
| Description | Elevation at the bottom of the inside portion of the pipe, at the downstream point |  |  |

L_PIPE16 – Pipe From

| Database Name | PIPE_FROM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Examples | (ID of the upstream fixed asset) |  |  |
| Description | The ID (*_ORID) of the asset from which the pipe flows |  |  |

L_PIPE17 – Pipe To

| Database Name | PIPE_TO |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Examples | (ID of the downstream fixed asset) |  |  |
| Description | The ID (*_ORID) of the asset to/toward which the pipe flows |  |  |

L_PIPE18 – Pipe Slope

| Database Name | PIPE_SLOPE |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (Whole number representing percent of slope, e.g. ‘11’ not ‘0.11’) |  |  |
| Description | Percent slope of the pipe, expressed as a whole number. Percent is calculated as: [(Rise/Run)*100 = Percent Slope] (Rise divided by run) multiplied by 100 = Percent Slope |  |  |

L_PIPE19 – Pipe Casing

| Database Name | PIPE_CASE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes, No, Unknown |  |  |
| Description | Flag to indicate if pipe has a casing or other protective container present |  |  |

L_PIPE20 – Pipe Horizontal Datum

| Database Name | PIPE_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

L_PIPE21 – Pipe Vertical Datum

| Database Name | PIPE_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

L_PIPE22 – Pipe General Location

| Database Name | PIPE_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 NW ½ of T31N R23W S17 121 7th Place East, Saint Paul, MN, 55101 210’ south of intersection of Smith Road and US HWY 52 44.957459, -93.277684 Intersection of W 26th Street and Blaisdell Avenue |  |  |
| Description | Data creator can provide general location information in the form of, PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

L_PIPE23 – Pipe As-Built Drawing Link

| Database Name | PIPEABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

L_PIPE24 – Pipe As-Built Drawing Document

| Database Name | PIPEABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing |  |  |

L_PIPE25 – Pipe Status

| Database Name | PIPE_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the pipe |  |  |

L_PIPE26 – Pipe Status Date

| Database Name | PIPE_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of last status assessment of the physical pipe |  |  |

L_PIPE27 – Pipe Installation Date

| Database Name | PIPE_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the physical pipe |  |  |

L_PIPE28 – Pipe Modification Date

| Database Name | PIPE_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the physical pipe |  |  |

L_PIPE30 – Pipe Condition

| Database Name | PIPE_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition” “Pipe exposed due to erosion” “Poor” |  |  |
| Description | PIPE_COND is provided as a 150-character field for written descriptions |  |  |

L_PIPE30 – Pipe Condition Date

| Database Name | PIPE_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last condition assessment of the physical pipe |  |  |

L_PIPE31 – Pipe Maintenance Agreement Number

| Database Name | PIPE_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Document ID of the agreement between agencies for the maintenance of the physical pipe |  |  |

L_PIPE32 – Pipe Easement

| Database Name | PIPE_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes, No, Unknown |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

L_PIPE33 – Pipe Consequence of Failure Rating

| Database Name | PIPE_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of pipe asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

L_PIPE34 – Pipe Probability of Failure Rating

| Database Name | PIPE_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of pipe asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

L_PIPE35 – Pipe Criticality to System

| Database Name | PIPE_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

L_PIPE36 – Pipe Ownership Type

| Database Name | PIPE_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the pipe |  |  |

L_PIPE37 – Pipe Ownership Name

| Database Name | PIPE_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the pipe |  |  |

L_PIPE38 – Pipe Maintenance Authority Type

| Database Name | PIPE_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the pipe |  |  |

L_PIPE39 – Pipe Maintenance Authority Name

| Database Name | PIPE_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Dakota County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the pipe |  |  |

L_PIPE40 – Pipe Data Producer/Source Type

| Database Name | PIPE_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

L_PIPE41 – Pipe Data Producer/Source Name

| Database Name | PIPE_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

L_PIPE42 – Pipe Date Data Modified

| Database Name | PIPE_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the pipe |  |  |

L_PIPE43 – Pipe Data Producer/Source Name

| Database Name | PIPE_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the pipe (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

L_PIPE44 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory name where the physical pipe is located; |  |  |

L_PIPE45 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = unique GNIS code for City of Eagan 02394198 = unique GNIS code for City of Bloomington |  |  |
| Description | Eight-digit GNIS code representing the municipal unit (CTU = city, township, unorganized territory) where the asset is located; as the leading zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT); GNIS is the Geographic Names Information System code developed by the U.S. Geological Survey and the U.S Board on Geographic Names to promote standardization of names of features with a unique number-based code. Helpful links: https://www.usgs.gov/core-science-systems/ngp/board-on-geographic-names https://www.usgs.gov/core-science-systems/ngp/board-on-geographic-names/domestic-names https://en.wikipedia.org/wiki/Geographic_Names_Information_System |  |  |

L_PIPE46 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 053 |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the pipe is located; because a linear feature may cross a municipal and/or county boundary, the general rule is that if over half the feature (>50%) is in one jurisdiction, favor that jurisdiction as the location rather than splitting the line at the boundary |  |  |

L_PIPE47 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical pipe is located; ; because a linear feature may cross a municipal and/or county boundary, the general rule is that if over half the feature (>50%) is in one jurisdiction, favor that jurisdiction as the location rather than splitting the line at the boundary |  |  |

L_PIPE48 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Examples | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

L_PIPE49 – Pipe Comments

| Database Name | PIPE_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Unsure if this pipe is actually located on the Johnson Property” “Pipe is damaged, needs to be replaced based on field inspection May 2019” “Polyline from data source digitized in wrong direction” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of the pipe feature |  |  |

Channel Components

L_CHAN1 – Channel ID

| Database Name | CHAN_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | C117-14-4 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

L_CHAN2 – Channel Federated ID

| Database Name | CHAN_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202- C117-14-4 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-C117-14-4

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

C117-14-4 = Example of the locally designated ID for the feature

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

L_CHAN3 – AUID

| Database Name | CHAN_AUID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 12 | Domain | AUID |
| Example | 04010201-A79 07040008-871 |  |  |
| Description | Assessment Unit ID (to be changed to WIDs – Water Unit IDs) ID for streams, rivers, ditches and other types of open channels |  |  |

Consideration of potentially removing this attribute from the MSWGP v. 0.5. dataset;

AUID/WIDs are not permanent IDs, and the extent of a reach is subject to change over time, influenced by factors such as water quality standards, channelization or restoration work, hydrological influence, and other reasons that influence how the Minnesota Pollution Control Agency (MPCA) conducts water quality work. If the extent of a reach is changed, the WID number will also change. Historical associations are maintained by MPCA. Streams without a specific agency interest associated have an ID of '*HUC8*-999' assigned to them. As the MPCA programs associate stations or attributes with these un-specified waters, new WIDs will be continually assigned (https://gisdata.mn.gov/dataset/water-current-stream-wids).

L_CHAN4 – Channel Type

| Database Name | CHAN_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 30 | Domain | ChannelType |
| Examples | Ditch, Trench, Aqueduct, Emergency Overflow, Swale, Stream, Spillway, Lined Channel, Natural Channel, Other, Unknown |  |  |
| Description | Generalized channel type description |  |  |

L_CHAN5 – Channel Design Depth

| Database Name | CHAN_DD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (insert value in feet) |  |  |
| Description | Channel design depth in feet (taken from as-built drawings) |  |  |

L_CHAN6 – Channel Width

| Database Name | CHAN_WD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (insert value in feet) |  |  |
| Description | Channel width measured at surface in feet (or from as-built drawings) |  |  |

L_CHAN7 – Channel Length

| Database Name | CHAN_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (insert value in feet) |  |  |
| Description | Channel length measured in feet (taken from as-built drawings) |  |  |

L_CHAN8 – Channel Lining

| Database Name | CHAN_LIN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 24 | Domain | ChannelLining |
| Examples | “Vegetation, high” “Armored, concrete” “Armored, concrete, bottom only” “Unknown” (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |
| Description | A description of the lining material of the channel or ditch |  |  |

L_CHAN9 – Channel Shape

| Database Name | CHAN_SHP |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 24 | Domain | ChannelShape |
| Examples | Natural, Irregular, Triangular-Symmetrical, Triangular-Asymmetrical, Trapezoidal, Segmental, Segmental-Two Stage, Parabolic, Rectangular-Wide, Rectangular-Narrow, Other, Unknown |  |  |
| Description | The cross-sectional shape of a channel or ditch |  |  |

L_CHAN10 – Channel From

| Database Name | CHAN_FROM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Examples | (ID of the upstream fixed asset) |  |  |
| Description | The ID (*_ORID) of the asset from which the channel flows |  |  |

L_CHAN11 – Channel To

| Database Name | CHAN_TO |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Examples | (ID of the downstream fixed asset) |  |  |
| Description | The ID (*_ORID) of the asset to/toward which the channel flows |  |  |

L_CHAN12 – Channel Slope

| Database Name | CHAN_SLOPE |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (Whole number representing percent of slope, e.g. ‘11’ not ‘0.11’) |  |  |
| Description | Percent slope of the pipe, expressed as a whole number Percent is calculated as: [(Rise/Run)*100 = Percent Slope] |  |  |

L_CHAN13 – Channel Horizontal Datum

| Database Name | CHAN_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

L_CHAN14 – Channel Vertical Datum

| Database Name | CHAN_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

L_CHAN15 – Channel General Location

| Database Name | CHAN_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 NW ½ of T31N R23W S17 14005 South Range Line Road 1100’ south of intersection 110th Street and 190th Avenue, W of Strathcona 44.957459, -93.277684 Intersection of Range Line Road and Highway 23 |  |  |
| Description | Data creator can provide general location information in the form of, PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

L_CHAN16 – Channel As-Built Drawing Link

| Database Name | CHANABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

L_CHAN17 – Channel As-Built Drawing Document

| Database Name | CHANABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing |  |  |

L_CHAN18 – Channel Status

| Database Name | CHAN_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the channel |  |  |

L_CHAN19 – Channel Status Date

| Database Name | CHAN_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of last status assessment of the physical channel |  |  |

L_CHAN20 – Channel Installation Date

| Database Name | CHAN_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the physical channel |  |  |

L_ CHAN21 – Channel Modification Date

| Database Name | CHAN_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the physical channel |  |  |

L_ CHAN22 – Channel Condition

| Database Name | CHAN_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition” “Heavy erosion on western side” “Channel blocked with debris” |  |  |
| Description | CHAN_COND is provided as a 150-character field for written descriptions |  |  |

L_ CHAN23 – Channel Condition Date

| Database Name | CHAN_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the physical channel |  |  |

L_ CHAN24 – Channel Maintenance Agreement Number

| Database Name | CHAN_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Number of document ID of agreement between agencies for the maintenance of the physical channel |  |  |

L_ CHAN25 – Channel Easement

| Database Name | CHAN_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = there is an easement associated with the feature No = it has been confirmed that there is no easement Unknown = it is unknown if an easement is present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

L_CHAN26 – Channel Consequence of Failure Rating

| Database Name | CHAN_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of channel asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

L_CHAN27 – Channel Probability of Failure Rating

| Database Name | CHAN_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of channel asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

L_CHAN28 – Channel Criticality to System

| Database Name | CHAN_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

L_CHAN29 – Channel Ownership Type

| Database Name | CHAN_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the channel |  |  |

L_CHAN30 – Channel Ownership Name

| Database Name | CHAN_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the channel |  |  |

L_CHAN31 – Channel Maintenance Authority Type

| Database Name | CHAN_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the channel |  |  |

L_CHAN32 – Channel Maintenance Authority Name

| Database Name | CHAN_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the channel |  |  |

L_CHAN33 – Channel Data Producer/Source Type

| Database Name | CHAN_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

L_CHAN34 – Channel Data Producer/Source Name

| Database Name | CHAN_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

L_CHAN35 – Channel Date Data Modified

| Database Name | CHAN_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the channel |  |  |

L_CHAN36 – Channel Data Producer/Source Name

| Database Name | CHAN_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the channel (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

L_CHAN37 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory name where the physical channel is located. Because a linear feature may cross a municipal and/or county boundary, the general rule is that if over half the feature (>50%) is in one jurisdiction, favor that jurisdiction as the location rather than splitting the line at the boundary |  |  |

L_CHAN38 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 02394198 |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located. As the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT) |  |  |

L_CHAN39 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the channel is located, please see links on page 25 for additional information; |  |  |

L_CHAN40 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical channel is located, because a linear feature may cross a municipal and/or county boundary, the general rule is that if over half the feature (>50%) is in one jurisdiction, favor that jurisdiction as the location rather than splitting the line at the boundary |  |  |

L_CHAN41 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

L_CHAN42 – Channel Comments

| Database Name | CHAN_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Unsure if this channel is actually located on the Anderson Property” “Channel is heavily eroded and damaged from ATVs” “Polyline from data source digitized in wrong direction” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of the channel feature |  |  |

Artificial Path Components

L_APATH1 – Artificial Path ID

| Database Name | APA_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | AP-XH-00088 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

L_APATH2 – Artificial Path Federated ID

| Database Name | APA_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-AP-XH-00088 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-AP-XH-00088

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

AP-XH-00088 = Example of the locally-designated unique ID for the feature

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

L_APATH3 – Artificial Path Type

| Database Name | APA_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 45 | Domain | ArtificialPathType |
| Examples | “Water connector” |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

L_APATH4 – Artificial Path Length

| Database Name | APA_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | Mandatory |
| Width | Default | Domain | (no domain) |
| Examples | (insert value in feet) |  |  |
| Description | Artificial path length measured in feet (calculated in GIS) |  |  |

L_APATH5 – Artificial Path From

| Database Name | APA_FROM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Examples | (ID of the upstream fixed asset) |  |  |
| Description | The ID (*_ORID) of the asset from which the artificial path flows |  |  |

L_APATH6 – Artificial Path To

| Database Name | APA_TO |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Examples | (ID of the downstream fixed asset) |  |  |
| Description | The ID (*_ORID) of the asset to/toward which the artificial path flows |  |  |

L_APATH7 – Artificial Path Slope

| Database Name | APA_SLOPE |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | (Whole number representing percent of slope, e.g. ‘11’ not ‘0.11’) |  |  |
| Description | Percent slope of the artificial path, expressed as a whole number Percent is calculated as: [(Rise/Run)*100 = Percent Slope] |  |  |

L_APATH8 – Artificial Path Status

| Database Name | APA_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples* | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the artificial path (as applicable) |  |  |

*Obviously, several of these options are not applicable to an artificial path feature

L_APATH9 – Artificial Path Data Producer/Source Type

| Database Name | APA_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

L_APATH10 – Artificial Path Data Producer/Source Name

| Database Name | APA_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

L_APATH11 – Artificial Path Date Data Modified

| Database Name | APA_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the path |  |  |

L_APATH12 – Artificial Path Producer/Source Name

| Database Name | APA_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the artificial; this can be an individual, department, agency, etc. |  |  |

L_APATH13 –– CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory where the artificial path is located; |  |  |

L_APATH14 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 02394198 |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the artificial path is located As the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT) |  |  |

L_APATH15 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the artificial path is located, because a linear feature may cross a municipal and/or county boundary, the general rule is that if over half the feature (>50%) is in one jurisdiction, favor that jurisdiction as the location rather than splitting the line at the boundary |  |  |

_APATH16 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the artificial path is located |  |  |

L_CHAN17 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

L_APATH18 – Artificial Path Comments

| Database Name | APA_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Created path to replicate known overland flow” “Path used for the known thalweg* derived from LIDAR” “Water connector through Smith Lake for flow modeling” |  |  |
| Description | General field for text comments related to any descriptive aspect of the artificial path feature |  |  |

*What the heck is a ‘thalweg’ anyway?

“Thalweg” is a term used in geography and geomorphology and often in legal proceedings where a watercourse forms a legal boundary between two jurisdictions. Translated from the original German, ‘thalweg’ literally means ‘valley way’ and is the term used for the path of the lowest elevation in a waterway, water course or valley. In the example below, the stream centerline is in light blue, while the thalweg (deepest water course) is shown in orange.

Basin Components

P_BASN.1 – Basin Unique Identifier

| Database Name | BASN_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | CEB-100020003-009 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_BASN.2 – Basin Federated ID

| Database Name | BASN_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-CEB-100020003-009 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-CEB-100020003-009

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

CEB-100020003-009= Example of the locally-designated unique ID for the basin feature

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

P_BASN.3 – Basin Local Parcel ID

| Database Name | BASN_PID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 36 | Domain | (no domain) |
| Example |  |  |  |
| Description | Unique locally assigned ID of the parcel on which the basin point is located. |  |  |

The purpose of the ‘Local Parcel ID’ is provide the ability to link the basin point to the parcel of land on which it is found; by embedding the parcel ID into the basin point, this affords the opportunity for additional relational database linkages and analysis to be made. It is acknowledged and accepted that basin extent (e.g. the polygon representation of the basin) can potentially extend over numerous parcels, or, in some cases a basin point may be in a right of way or other area where there no parcels IDs are assigned. In these instances, it is up to the data creator make a best fit determination relevant and usable to their circumstance.

P_BASN.4 – Basin DNR Lake ID

| Database Name | BASN_DNRID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 10 | Domain | (no domain) |
| Example | 27112900 |  |  |
| Description | A unique 8-digit identifier for each lake polygon as assigned by the Minnesota Department of Natural Resources (DNR). The value of this field is the DNR Division of Waters lake identification number if one has been assigned. Otherwise, the Lake ID is a unique sequential number. |  |  |

A unique 8-digit identifier for each lake polygon as assigned by the Minnesota Department of Natural Resources.

More information is available at the link: https://www.dnr.state.mn.us/maps/compass/index.html

P_BASN.5 – Basin Public Waters Index (PWI) ID

| Database Name | BASN_PWI |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 12 | Domain | (no domain) |
| Example | 2P, 1084P, 1085P, 1086P |  |  |
| Description | A unique ID for bodies of water on the Public Waters Index that meet the criteria of Mn. Stat. 103G.005 and have been mapped under Mn. Stat. 103G.201 |  |  |

Any basin that has been assigned PWI inventory ID number. The Minnesota Department of Natural Resources is presently in the process of creating geospatial data representing PWI waters. Original paper maps with ID numbers assigned to PWI water are available at the link:

https://www.dnr.state.mn.us/waters/watermgmt_section/pwi/maps.html

P_BASN.6 – Basin Name

| Database Name | BASN_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | (no domain) |
| Example | Black Dog Lake, Cedar Pond |  |  |
| Description | Common name of the basin |  |  |

P_BASN.7 – Basin Type

| Database Name | BASN_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 30 | Domain | BasinType |
| Example | Lake, Pond, Wetland, Constructed Wetland, Culvert (centroid), Detention, Impoundment, Filtration with underdrain, Filtration without underdrain, Other, Unknown |  |  |
| Description | Type of basin |  |  |

Within limnology and water resources management, the term ‘basin’ is a generalized term, but there are many basin types, each with specific definitions. The difference between a ‘lake’ and a ‘pond’ for example is the presence (in a lake) of an ‘aphotic’ zone (i.e. an area where sunlight cannot reach the bottom) or its absence (as in a pond). Visual analysis may not be enough to correctly attribute each basin feature, additional research may be needed to determine the correct category for each water body represented.

The water body shown at right is an example of a constructed wetland however it appears as if it were a naturally occurring basin feature.

The table at the top of the next page provides the general description and definition of the various basin types represented in the v. 0.5 of the stormwater geodata standard as offered for public review.

| Basin Value | General Definition |
| --- | --- |
| Lake | Water body which contains an 'aphotic' zone (i.e. area where sunlight cannot reach the bottom) |
| Pond | Water body which lacks an 'aphotic' zone (i.e. sunlight can reach all parts of the water body bottom) |
| Wetland | A wetland that occurs in its natural or near-natural state |
| Constructed Wetland | A wetland that occurs as the result of human construction |
| Culvert, centroid | A point that represents the centroid of a line segment representing a culvert |
| Detention | An excavated area or feature adjacent to a waterbody to store water for a limited period of time |
| Impoundment | A body of water formed by a dam or other structure |
| Filtration with underdrain | A filtration system with an underdrain |
| Filtration without underdrain | A filtration system without an underdrain |
| Other | A feature that does not fall into another clear definition category |
| Unknown | Feature type is unknown |

Example of a detention

(dry pond style) feature:

Example of a pond

(constructed) feature:

Example of a filtration basin

(constructed) feature:

P_BASN.8 – Basin Depth

| Database Name | BASN_DEPTH |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of depth in feet) |  |  |
| Description | Maximum depth of the basin in feet (as per as-built drawings or as known if a natural feature) |  |  |

P_BASN.9 – Basin Area

| Database Name | BASN_AREA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of depth in acres) |  |  |
| Description | Surface of the basin in acres (as per as-built drawings or as known if a natural feature) |  |  |

P_BASN.10 – Basin Design Volume

| Database Name | BASN_VLD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of depth in acre-feet) |  |  |
| Description | Volume of water the basin was designed to hold (if constructed) or holds naturally |  |  |

P_BASN.11 – Basin Design Normal Elevation

| Database Name | BASN_NELV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet-above-sea-level) |  |  |
| Description | Normal elevation of the basin above sea level |  |  |

P_BASN.12 – Basin Design Flood Stage Elevation

| Database Name | BASN_FELV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet-above-sea-level) |  |  |
| Description | 100-year flood elevation based on local datum |  |  |

P_BASN.13 – Basin Design Bottom Elevation

| Database Name | BASN_BELV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet-above-sea-level) |  |  |
| Description | Bottom elevation of the basin (as per as-built drawings or as known if a natural feature) |  |  |

P_BASN.14 – Basin Overflow Elevation

| Database Name | BASN_OELV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet-above-sea-level) |  |  |
| Description | Elevation at which overflow of the basin takes place (as per as-built drawings or as known if a natural feature) |  |  |

P_BASN.15 – Basin Ordinary High-water Level

| Database Name | BASN_OHWL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet-above-sea-level) |  |  |
| Description | Ordinary high-water level (as defined by MN Stat. 103G.005, Subd. 14) |  |  |

P_BASN.16 – Basin Horizontal Datum

| Database Name | BASN_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_BASN.17 – Basin Vertical Datum

| Database Name | BASN_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_BASN.18 – Basin General Location

| Database Name | BASN_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 NW ½ of T31N R23W S17 44.957459, -93.277684 Intersection of Sections 1, 2, 11 and 12 within T29N R29E |  |  |
| Description | Data creator can provide general location information in the form of, PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

P_BASN.19 – Basin Origin

| Database Name | BASN_ORIG |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 30 | Domain | BasinOrigin |
| Examples | Natural Natural-modified Constructed Restored to original condition Restored to modified condition Unknown Other |  |  |
| Description | Indication of the origin status of the basin (naturally occurring or the result of human action) |  |  |

P_BASN.20 – Basin 303(d) Impairment Status

| Database Name | BASN_303D |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 12 | Domain | BasinImpaired |
| Examples | Impaired, Not impaired, Prior impairment, Maintenance At risk, Not applicable, Unknown |  |  |
| Description | Indication if a water body (lake, stream/river segment) is currently listed on the State of Minnesota impaired/threatened waters list |  |  |

This attribute is in reference to Section 303(d) of the Federal Clean Water Act for impaired waters and total maximum daily loads (TMDLs). The U. S. Environmental Protection Agency has granted to the Minnesota Pollution Control Agency the role of monitoring and reporting on impaired waters.

Helpful links and resources which reference water impairments under Section 303(d)

https://www.epa.gov/tmdl

https://www.pca.state.mn.us/water/minnesotas-impaired-waters-list

https://www.pca.state.mn.us/water/impaired-waters-viewer-iwav

P_BASN.21 – Basin As-Built Drawing Link

| Database Name | BASNABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

P_BASN.22 – Basin As-Built Drawing Document

| Database Name | BASNABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | The document number, ID number, or reference number of the original as-built drawing of the basin |  |  |

P_BASN.23 – Basin Status

| Database Name | BASN_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the basin |  |  |

P_BASN.24 – Basin Status Date

| Database Name | BASN_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of last status assessment of the physical basin |  |  |

P_BASN.25 – Basin Installation Date

| Database Name | BASN_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the physical basin |  |  |

P_BASN.26 – Basin Modification Date

| Database Name | BASN_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the physical basin |  |  |

P_BASN.27 – Basin Condition

| Database Name | BASN_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition when inspected September 2, 2019” “Heavy erosion on western side” |  |  |
| Description | BASN_COND is provided as a general 150-character field for written descriptions about the observed or known condition of the basin. |  |  |

P_BASN.28 – Basin Condition Date

| Database Name | BASN_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the when the BASN_COND comment was entered/relevant |  |  |

P_BASN.29 – Basin Maintenance Agreement Number

| Database Name | BASN_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Number of document ID of agreement between agencies for the maintenance of the physical basin |  |  |

P_BASN.30 – Basin Easement

| Database Name | BASN_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = an easement is present No = no easement is present Unknown = it is unknown if there is an easement present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

P_BASN.31 – Basin Consequence of Failure Rating

| Database Name | BASN_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_BASN.32 – Basin Probability of Failure Rating

| Database Name | BASN_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_BASN.33 – Basin Criticality to System

| Database Name | BASN_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_BASN.34 – Basin Ownership Type

| Database Name | BASN_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the basin |  |  |

P_BASN.35 – Basin Ownership Name

| Database Name | BASN_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the basin |  |  |

P_BASN.36 – Basin Maintenance Authority Type

| Database Name | BASN_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the basin |  |  |

P_BASN.37 – Basin Maintenance Authority Name

| Database Name | BASN_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the basin |  |  |

P_BASN.38 – Basin Data Producer/Source Type

| Database Name | BASN_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_BASN.39 – Basin Data Producer/Source Name

| Database Name | BASN_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_BASN.40 – Basin Date Data Modified

| Database Name | BASN_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the basin |  |  |

P_BASN.41 – Basin Data Source

| Database Name | BASN_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the basin (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_BASN.42 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory name where the basin is physically is located; if a basin centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the basin in |  |  |

P_BASN.43 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_BASN.44 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the basin is located, please see links on page 25 for additional information; |  |  |

P_BASN.45 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical basin is located |  |  |

P_BASN.46 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_BASN.47 – Basin Comments

| Database Name | BASN_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “This basin overflows every April-May” “Basin needs maintenance, east wall is eroded” “Basin centroid represents the location pre-flood of 2012” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of the basin feature |  |  |

Hydraulic Control Structures Components

P_HCS.1 – Hydraulic Control Structure Unique Identifier

| Database Name | HCS_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | 103-HCHCS-0041-0177 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner; Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_HCS.2 – Hydraulic Control Structure Federated ID

| Database Name | HCS_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-103-HCHCS-0041-0177 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-103-HCHCS-0041-0177

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

103-HCHCS-0041-0177 Example of the locally-designated unique ID for the structure

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

P_HCS.3 – Hydraulic Control Structure Type

| Database Name | HCS_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 30 | Domain | HCSType |
| Example | Dam, deck drain, detention tank, ditch block, energy dissipator, diversion chamber, diversion point, diverter, flow restrictor, outlet control structure, etc. (for complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |
| Description | Type of hydraulic control structure |  |  |

There are numerous kinds of hydraulic control structures listed in domains associated with the MSWGP v. 0.5 draft standard. The MSWGP working group recognizes the limits of a GIS system to be able to fully represent all the various details of each potential kind of structure. With most of the structures, this standard intends mainly to be able to accurately document the position/location of these assets, their essential measurements, their function and their ownership and other common details which would benefit the data users and hopefully provide value to the engineering and asset management data user community.

P_HCS.4 – Hydraulic Control Structure Length

| Database Name | HCS_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Length in feet of the structure |  |  |

P_HCS.5 – Hydraulic Control Structure Width

| Database Name | HCS_WID |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in inches) |  |  |
| Description | Width in inches of the structure |  |  |

P_HCS.6 – Hydraulic Control Structure Height or Mean Depth

| Database Name | HCS_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Height or mean depth of the structure |  |  |

P_HCS.7 – Hydraulic Control Structure Design Volume

| Database Name | HCS_VLD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of depth in cubic-feet) |  |  |
| Description | Volume of water in cubic feet the structure was designed to hold (if constructed) or holds naturally; |  |  |

P_HCS.8 – Hydraulic Control Structure Invert Elevation

| Database Name | HCS_IELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of depth in feet above mean-sea-level) |  |  |
| Description | The elevation of the bottom of the inside portion of the outlet of the structure, in units of feet above mean sea level; |  |  |

P_HCS.9 – Hydraulic Control Structure Rim Elevation

| Database Name | HCS_RELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Rim elevation (manholes); elevation of the center of the manhole lid measured from its top in feet above mean sea level |  |  |

P_HCS.10 – Hydraulic Control Structure Bottom Elevation

| Database Name | HCS_BELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Bottom elevation of structure (differentiated from the invert elevation) |  |  |

P_HCS.11 – Hydraulic Control Structure Rotation

| Database Name | HCS_ROTAT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (value = angle of rotation for cartographic symbol [azimuthal; north = 0°]) |  |  |
| Description | Recommended angle of rotation for cartographic symbol; this field is used to provide a value for the preferred rotation of the point symbol for proper mapping display; the 360° azimuthal system is used (clockwise rotation) (e.g.: north= 0°, east = 90°, south= 180°, west = 270°) |  |  |

P_HCS.12 – Hydraulic Control Structure Horizontal Position Accuracy Value

| Database Name | HCS_HPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_HCS.13) |  |  |
| Description | Indicator of the accuracy of x and y value of the structure |  |  |

P_HCS.13 – Hydraulic Control Structure Horizontal Position Unit

| Database Name | HCS_HPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Hydraulic Control Structure Horizontal Position Accuracy Value |  |  |

P_HCS.14 – Hydraulic Control Structure Vertical Position Accuracy Value

| Database Name | HCS_VPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_HCS.15) |  |  |
| Description | Indicator of the accuracy of the z value of the structure |  |  |

P_HCS.15 – Hydraulic Control Structure Vertical Position Unit

| Database Name | HCS_VPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Hydraulic Control Structure Vertical Position Accuracy Value |  |  |

Examples of the prior four attributes within the context of data in the standard:

| HCS_IELEV | HCS_RELEV | HCS_BELEV | HCS_ROTAT | HCS_HPAV | HCS_HPU | HCS_VPAV | HCS_VPU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 879.1 | 882.1 | 879.1 | 90.0 | 2.1 | Feet | 1.4 | Feet |
| 880.4 | 883.9 | 880.4 | 0.0 | 0.6 | Meters | 0.3 | Meters |
| 890.7 | 893.7 | 890.7 | 60.0 | 0.75 | Meters | 0.75 | Meters |
| 877.8 | 890.0 | 877.8 | 90.0 | 1.8 | Feet | 3.0 | Feet |

Note: Ideally, all horizontal and vertical accuracy indicators would be in the same unit of measure, however; as this draft standard is offered primarily as a functional transfer standard, the ability to pool data together from different measures into one body of data was initially seen as desirable, with the end-user needing to perform the needed calculations to standardize the data as needed.

This remains a point open to debate and suggestion as this draft standard is reviewed by the stakeholder community. We welcome thoughts and ideas on how to improve this attribute in the standard.

P_HCS.17 – Hydraulic Control Structure Horizontal Datum

| Database Name | HCS_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_HCS.18 – Hydraulic Control Structure Vertical Datum

| Database Name | HCS_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_HCS.18 – Hydraulic Control Structure General Location

| Database Name | HCS_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | 1119 22nd Avenue NE SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 NW ½ of T31N R23W S17 44.957459, -93.277684 55’ NW of intersection of 11th Street SW and Plymouth Avenue |  |  |
| Description | Data creator can provide general location information in the form of PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location; |  |  |

P_HCS.19 – Hydraulic Control Structure Drawing Link

| Database Name | HCS_ABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

P_HCS.20 – Hydraulic Control Structure Document

| Database Name | HCS_ABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing of the structure |  |  |

P_HCS.21 – Hydraulic Control Structure Status

| Database Name | HCS_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the hydraulic control structure |  |  |

P_HCS.22 – Hydraulic Control Structure Status Date

| Database Name | HCS_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of last status assessment of the structure |  |  |

P_HCS.23 – Hydraulic Control Structure Installation Date

| Database Name | HCS_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the structure |  |  |

P_HCS.24 – Hydraulic Control Structure Modification Date

| Database Name | HCS_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the structure |  |  |

P_HCS.25 – Hydraulic Control Structure Condition

| Database Name | HCS_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition when inspected in spring 2018” “Damaged from flooding” |  |  |
| Description | A 150-character field for subjective written descriptions |  |  |

P_HCS.26 – Hydraulic Control Structure Condition Date

| Database Name | HCS_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last known condition assessment |  |  |

P_HCS.27 – Hydraulic Control Structure Maintenance Agreement Number

| Database Name | HCS_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Document ID of agreement between agencies for the maintenance of the structure |  |  |

P_HCS.28 – Hydraulic Control Structure Easement

| Database Name | HCS_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = an easement is present No = no easement is present Unknown = it is unknown if there is an easement present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

P_HCS.29 – Hydraulic Control Structure Consequence of Failure Rating

| Database Name | HCS_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_HCS.30 – Hydraulic Control Structure Probability of Failure Rating

| Database Name | HCS_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_HCS.31 – Hydraulic Control Structure Criticality to System

| Database Name | HCS_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_HCS.32 – Hydraulic Control Structure Ownership Type

| Database Name | HCS_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the basin |  |  |

P_HCS.33 – Hydraulic Control Structure Ownership Name

| Database Name | HCS_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the structure |  |  |

P_HCS.34 – Hydraulic Control Structure Maintenance Authority Type

| Database Name | HCS_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the structure |  |  |

P_HCS.35 – Hydraulic Control Structure Maintenance Authority Name

| Database Name | HCS_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the structure |  |  |

P_HCS.36 – Hydraulic Control Structure Holds Water

| Database Name | HCS_HOLDS |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 10 | Domain | YesNoUnknown |
| Examples | Yes, No, Unknown |  |  |
| Description | Flag indicating if the structure holds water or not |  |  |

P_HCS.37 – Hydraulic Control Structure Infiltration Rate

| Database Name | HCS_INFIL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value for infiltration rate [inches/hour]) |  |  |
| Description | Rate of infiltration through the bottom of an infiltration device in inches per hour; |  |  |

P_HCS.38 – Hydraulic Control Structure Contributing Drainage Area

| Database Name | HCS_CDA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in acres) |  |  |
| Description | Surface area that discharges to the structure, measured in acres; |  |  |

P_HCS.39 – Hydraulic Control Structure Data Producer/Source Type

| Database Name | HCS_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_HCS.40 – Hydraulic Control Structure Data Producer/Source Name

| Database Name | HCS_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_HCS.41 – Hydraulic Control Structure Date Data Modified

| Database Name | HCS_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the structure |  |  |

P_HCS.42 – Hydraulic Control Structure Data Source

| Database Name | HCS_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the structure (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_HCS.43 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory name where the structure is physically is located; if a structure centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the structure in; |  |  |

P_HCS.44 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_HCS.45 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the structure is located, please see links on page 25 for additional information; |  |  |

P_HCS.46 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical structure is located |  |  |

P_HCS.47 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_HCS.48 – Hydraulic Control Structure Comments

| Database Name | HSC_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Needs maintenance or replacement, north wall cracked” “Centroid represents the location pre-flood of 2012” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of the basin feature; |  |  |

Pollution Control Structures Components

Components

P_PCS.1 – Pollution Control Structure Unique Identifier

| Database Name | PCS_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | 77077-4-4410-0001 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_PCS.2 – Pollution Control Structure Unique Identifier Structure Federated ID

| Database Name | PCS_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-77077-4-4410-0001 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-77077-4-4410-0001

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

77077-4-4410-0001 = Example of the locally-designated unique ID for the structure

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

P_PCS.3 – Pollution Control Structure Type

| Database Name | PCS_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 30 | Domain | PCSType |
| Example | Dam, deck drain, detention tank, ditch block, energy dissipator, diversion chamber, diversion point, diverter, flow restrictor, outlet control structure, etc. (for complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |
| Description | Type of hydraulic control structure |  |  |

There are numerous kinds of pollution control structures listed in domains associated with the MSWGP v. 0.5 draft standard, the MSWGP working group recognizes the limits of a GIS system to be able to fully represent all the various details of each potential kind of structure. With most of the structures, this standard intends mainly to be able to accurately document the position/location of these assets, their essential measurements, their function and their ownership and other common details which would benefit the data users and hopefully provide value to the engineering and asset management data user community.

Input, ideas and suggestions from stakeholder review of the standard are welcome as to how to improve the Pollution Control Structure Type category of the standard.

P_PCS.4 – Pollution Control Structure Length

| Database Name | PCS_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Length in feet of the structure |  |  |

P_PCS.5 – Pollution Control Structure Width

| Database Name | PCS_WID |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in inches) |  |  |
| Description | Width in inches of the structure |  |  |

P_PCS.6 – Pollution Control Structure Height or Mean Depth

| Database Name | PCS_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Height or mean depth of the structure |  |  |

P_PCS.7 – Pollution Control Structure Design Volume

| Database Name | PCS_VLD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of depth in cubic-feet) |  |  |
| Description | Volume of water in cubic feet the structure was designed to hold (if constructed) or holds naturally; |  |  |

P_PCS.8 – Pollution Control Structure Invert Elevation

| Database Name | PCS_IELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of depth in feet above mean-sea-level) |  |  |
| Description | The elevation of the bottom of the inside portion of the outlet of the structure, in units of feet above mean sea level; |  |  |

>> Please see Figure 3 on on page 58

P_PCS.9 – Pollution Control Structure Rim Elevation

| Database Name | PCS_RELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Rim elevation (manholes); elevation of the center of the manhole lid measured from its top in feet above mean sea level |  |  |

P_PCS.10 – Pollution Control Structure Bottom Elevation

| Database Name | PCS_BELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Bottom elevation of structure (differentiated from the invert elevation) |  |  |

>> Please see Figure 4 on on page 59

P_PCS.11 – Pollution Control Structure Rotation

| Database Name | PCS_ROTAT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (angle of rotation for cartographic symbol [azimuthal; north = 0°]) |  |  |
| Description | Recommended angle of rotation for cartographic symbol; this field is used to provide a value for the preferred rotation of the point symbol for proper mapping display; the 360° azimuthal system is used (clockwise rotation) (e.g.: north= 0°, east = 90°, south= 180°, west = 270°) |  |  |

>> Please see Figure 5 on page 59

P_PCS.12 – Pollution Control Structure Horizontal Position Accuracy Value

| Database Name | PCS_HPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See also P_PCS.13) |  |  |
| Description | Indicator of the accuracy of x and y value of the structure |  |  |

P_PCS.13 – Pollution Control Structure Horizontal Position Unit

| Database Name | PCS_HPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Pollution Control Structure Vertical Position Accuracy Value |  |  |

P_PCS.14 – Pollution Control Structure Vertical Position Accuracy Value

| Database Name | PCS_VPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See also P_PCS.15) |  |  |
| Description | Indicator of the accuracy of the z value of the structure |  |  |

P_PCS.15 – Pollution Control Structure Vertical Position Unit

| Database Name | PCS_VPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Pollution Control Structure Vertical Position Accuracy Value |  |  |

P_PCS.16 – Pollution Control Structure Vertical Datum

| Database Name | PCS_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_PCS.17 – Pollution Control Structure Horizontal Datum

| Database Name | PCS_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_PCS.18 – Pollution Control Structure General Location

| Database Name | PCS_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | 1119 22nd Avenue NE SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 NW ½ of T31N R23W S17 44.957459, -93.277684 55’ NW of intersection of 11th Street SW and Plymouth Avenue |  |  |
| Description | Data creator can provide general location information in the form of PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location; |  |  |

P_PCS.19 – Pollution Control Structure Drawing Link

| Database Name | PCS_ABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing of the structure |  |  |

P_PCS.20 – Pollution Control Structure Document

| Database Name | PCS_ABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing of the structure |  |  |

P_PCS.21 – Pollution Control Structure Status

| Database Name | PCS_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the pollution control structure |  |  |

P_PCS.22 – Pollution Control Structure Status Date

| Database Name | PCS_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of last status assessment of the structure |  |  |

P_PCS.23 – Pollution Control Structure Installation Date

| Database Name | PCS_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the physical structure |  |  |

P_PCS.24 – Pollution Control Structure Modification Date

| Database Name | PCS_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the physical structure |  |  |

P_PCS.25 – Pollution Control Structure Condition

| Database Name | PCS_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition when inspected in spring 2018” “Damaged from flooding” |  |  |
| Description | A 150-character field for subjective, written descriptions of known or observed condition of the asset |  |  |

P_PCS.26 – Pollution Control Structure Condition Date

| Database Name | PCS_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last known condition assessment of the structure |  |  |

P_PCS.27 – Pollution Control Structure Maintenance Agreement Number

| Database Name | PCS_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Document ID of agreement between agencies for the maintenance of the physical asset |  |  |

P_PCS.28 – Pollution Control Structure Easement

| Database Name | PCS_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = an easement is present No = no easement is present Unknown = it is unknown if there is an easement present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

P_PCS.29 – Pollution Control Structure Consequence of Failure Rating

| Database Name | PCS_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_PCS.30 – Pollution Control Structure Probability of Failure Rating

| Database Name | PCS_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_PCS.31 – Pollution Control Structure Criticality to System

| Database Name | PCS_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_PCS.32 – Pollution Control Structure Ownership Type

| Database Name | PCS_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the structure |  |  |

P_PCS.33 – Pollution Control Structure Ownership Name

| Database Name | PCS_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the structure |  |  |

P_PCS.34 – Pollution Control Structure Maintenance Authority Type

| Database Name | PCS_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the structure |  |  |

P_PCS.35 – Pollution Control Structure Maintenance Authority Name

| Database Name | PCS_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the structure |  |  |

P_PCS.36 – Pollution Control Structure Holds Water

| Database Name | PCS_HOLDS |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 10 | Domain | YesNoUnknown |
| Examples | Yes, No, Unknown |  |  |
| Description | Flag indicating if the structure holds water or not |  |  |

P_PCS.37 – Pollution Control Structure Infiltration Rate

| Database Name | PCS_INFIL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value for infiltration rate [inches/hour]) |  |  |
| Description | Rate of infiltration through the bottom of an infiltration device in inches per hour; |  |  |

P_PCS.38 – Pollution Control Structure Contributing Drainage Area

| Database Name | PCS_CDA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in acres) |  |  |
| Description | Surface area that discharges to the structure, measured in acres; |  |  |

P_PCS.39 – Pollution Control Structure Data Producer/Source Type

| Database Name | PCS_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_PCS.40 – Pollution Control Structure Data Producer/Source Name

| Database Name | PCS_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_PCS.41 – Pollution Control Structure Date Data Modified

| Database Name | PCS_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the structure |  |  |

P_PCS.42 – Pollution Control Structure Data Source

| Database Name | PCS_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the structure (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_PCS.43 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory name where the structure is physically is located; if a structure centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the structure in; |  |  |

P_PCS.44 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_PCS.45 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the structure is located, please see links on page 25 for additional information; |  |  |

P_PCS.46 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical structure is located |  |  |

P_PCS.47 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_PCS.48 – Pollution Control Structure Comments

| Database Name | PSC_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Needs maintenance or replacement, north wall cracked” “Centroid represents the location pre-flood of 2012” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of the structure; |  |  |

Artificial Point Components

What is an artificial point? An artificial point is a point feature in the stormwater geodata that does not correspond to an actual physical feature on the landscape, however, it may serve to be useful for the work of modeling, tracking, or facilitating linkages to represent other virtual features in the dataset that correspond to flow analysis or maybe needed for engineering, modeling or asset management.

Within the MSWGP v. 0.5 schema, we have identified 5 ‘types’ of artificial points, these are

‘junction point’, ‘discharge point’, ‘centroid’ (e.g. it may be useful in some situations to have/maintain a centroid of a channel segment or pipe segment), ‘other’ and ‘unknown’. In the hypothetical example at right, a hydraulic modeler may wish to connect various parts of a stream system together. Artificial paths and artificial points can be created and placed to assist the modeler with more effective flow model development.

P_APNT.1 – Artificial Point ID

| Database Name | APT_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | ARTPT-0612-001 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the artificial point as used by its creator; Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_APNT.2 – Artificial Point Federated ID

| Database Name | APT_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202- ARTPT-0612-001 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example shown in P_APNT.2 we have a federated ID of:

2705300664202-ARTPT-0612-001

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

ARTPT-0612-001 = Example of the locally-designated unique ID for the feature

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

P_APNT.3 – Artificial Point Type

| Database Name | APT_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 45 | Domain | ArtificialPointType |
| Examples | Junction point Discharge point Centroid Other Unknown |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

P_APNT.4 – Artificial Point Status

| Database Name | APT_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples* | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the artificial point (as applicable) |  |  |

*The attributes of the global MSWGP v. 0.5 domain ‘Status’ have been applied to Artificial Point; obviously many of these options are not applicable, we welcome recommendations and suggestions for this domain.

P_APNT.5 – Artificial Point Elevation

| Database Name | APT_ELV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | Conditional |
| Width | Default | Domain | (no domain) |
| Examples | 683.4 (value indicating feet above sea level) |  |  |
| Description | Assigned elevation (in feet above mean sea level) for the artificial point feature |  |  |

P_APNT.6 – Artificial Point Data Producer/Source Type

| Database Name | APA_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_APNT.7 – Artificial Point Data Producer/Source Name

| Database Name | APT_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_APNT.8 – Artificial Point Date Data Modified

| Database Name | APT_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the feature |  |  |

P_APNT.9 – Artificial Point Producer/Source Name

| Database Name | APT_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the artificial point; this can be an individual, department, agency, etc. |  |  |

P_APNT.10 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 100 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory name where the artificial point is located; |  |  |

P_APNT.11 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the artificial point is located As the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT) |  |  |

P_APNT.12 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the artificial point is located |  |  |

P_APNT.13 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Stearns Hennepin |  |  |
| Description | Name of the county where the artificial point is located |  |  |

P_APNT.14 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_APNT.15 – Artificial Path Comments

| Database Name | APT_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Created point for known discharge point” “Point is the junction of two streams” “Point is where channel discharges at high-water mark of lake” |  |  |
| Description | General field for text comments related to any descriptive aspect of the artificial point feature |  |  |

Inlet Components

P_IN.1 – Inlet Unique Identifier

| Database Name | IN_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | 77456-01265 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_IN.2 – Inlet Federated Identifier

| Database Name | IN_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-77456-01265 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-77456-01265

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

77456-01265 = Example of the locally-designated unique ID for the structure

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

Inlet ‘Flag’ Attributes: The following ten components (P_IN.3 through P_IN.12) enable the data creator to ‘flag’ each inlet with the types general features it contains. A single inlet may be flagged with one or more of the following characteristics; it was determined that this approach enables the stormwater data creator to have maximum flexibility for categorizing and attributing the features.

P_IN.3 – Inlet Apron

| Database Name | IN_APRON |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if an apron conveying the stormwater from grade down to the inlet entrance is present; |  |  |

P_IN.4 – Inlet Curb Opening

| Database Name | IN_CURB |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if a drainage inlet is an opening in the roadway curb |  |  |

P_IN.5 – Inlet Deck Drain

| Database Name | IN_DECK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate a drainage inlet from a bridge deck or a scupper |  |  |

P_IN.6 – Inlet Drop

| Database Name | IN_DROP |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate a drainage inlet with a horizontal or nearly horizontal opening |  |  |

P_IN.7 – Inlet Flanking

| Database Name | IN_FLANK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate a drainage inlet placed on either side of an inlet at low point in a vertical curve to intercept debris as the slope decreases (acts as relief to the inlet at the lower point) |  |  |

P_IN.8 – Inlet Grate

| Database Name | IN_GRATE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate a drainage inlet composed of a grate in the roadway section or at the roadside low point or channel; |  |  |

P_IN.9 – Inlet Slotted

| Database Name | IN_SLOT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate a drainage inlet is composed of a continuous slot built into the top of a pipe which serves to intercept, collect and transport the flow. |  |  |

P_IN.10 – Inlet Trap

| Database Name | IN_TRAP |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if a drainage inlet is a trap |  |  |

P_IN.11 – Inlet Trench Drain

| Database Name | IN_TRDR |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if a drainage inlet is a trench drain |  |  |

P_IN.12 – Inlet Sump

| Database Name | IN_SUMP |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if a drainage inlet is a sump |  |  |

P_IN.13 – Inlet Shape

| Database Name | IN_SHAPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 14 | Domain | InletShape |
| Examples | Rectangle, Square, Circular, Trapezoid, Other, Unknown, Not applicable |  |  |
| Description | To provide a description of the predominant cross-sectional shape configuration of the inlet basic shapes of drains/inlet features (as applicable) |  |  |

P_IN.14 – Inlet Length

| Database Name | IN_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Length in feet of the structure |  |  |

P_IN.15 – Inlet Width

| Database Name | IN_WID |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in inches) |  |  |
| Description | Width in inches of the structure |  |  |

P_IN.16 – Inlet Height or Mean Depth

| Database Name | IN_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Height or mean depth of the structure |  |  |

P_IN.17 – Inlet Design Volume

| Database Name | IN_VLD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of depth in cubic-feet) |  |  |
| Description | Volume of water in cubic feet the structure was designed to hold (if constructed) or holds naturally; |  |  |

P_IN.18 – Inlet Invert Elevation

| Database Name | IN_IELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet above mean-sea-level) |  |  |
| Description | The elevation of the bottom of the inside portion of the outlet of the structure, in units of feet above mean sea level; |  |  |

P_IN.19 – Inlet Rim Elevation

| Database Name | IN_RELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Rim elevation (manholes); elevation of the center of the manhole lid measured from its top in feet above mean sea level |  |  |

P_IN.20 – Inlet Bottom Elevation

| Database Name | IN_BELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Bottom elevation of structure (differentiated from the invert elevation) |  |  |

P_IN.21 – Inlet Rotation

| Database Name | IN_ROTAT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (value = angle of rotation for cartographic symbol [azimuthal; north = 0°]) |  |  |

>> (Please see rotation example in Figure 5 on Page 59)

P_IN.22 – Inlet Horizontal Position Accuracy Value

| Database Name | IN_HPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_IN.23) |  |  |
| Description | Indicator of the accuracy of x and y value of the structure |  |  |

P_IN.23 – Inlet Horizontal Position Unit

| Database Name | IN_HPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Inlet Horizontal Position Accuracy Value |  |  |

P_IN.24– Inlet Vertical Position Accuracy Value

| Database Name | IN_VPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_IN.25) |  |  |
| Description | Indicator of the accuracy of the z value of the structure |  |  |

P_IN.25 – Inlet Vertical Position Unit

| Database Name | IN_VPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Inlet Vertical Position Accuracy Value |  |  |

P_IN.26 – Inlet Horizontal Datum

| Database Name | IN_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_IN.27 – Inlet Vertical Datum

| Database Name | IN_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_IN.28 – Inlet General Location

| Database Name | IN_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | 2022 42nd Avenue SW SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 44.957459, -93.277684 34’ SE of intersection of 15th Street SW and Connecticut Avenue |  |  |
| Description | Data creator can provide general location information in the form of PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

P_IN.29 – Inlet Drawing Link

| Database Name | IN_ABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

P_IN.30 – Inlet Document

| Database Name | IN_ABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing of the structure |  |  |

P_IN.31 – Inlet Structure Status

| Database Name | IN_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the structure |  |  |

P_IN.32 – Inlet Status Date

| Database Name | IN_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the status assessment of the structure |  |  |

P_IN.33 – Inlet Condition

| Database Name | IN_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition when inspected in spring 2018” “Damaged from flooding” |  |  |
| Description | A 150-character field for subjective written descriptions |  |  |

P_IN.34 – Inlet Condition Date

| Database Name | IN_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the condition description in P_IN.33 |  |  |

P_IN.35 – Inlet Installation Date

| Database Name | IN_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the structure |  |  |

P_IN.36 – Inlet Maintenance Agreement Number

| Database Name | IN_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Document ID of agreement between agencies for the maintenance of the structure |  |  |

P_IN.37 – Inlet Modification Date

| Database Name | IN_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the structure |  |  |

P_IN.38 – Inlet Easement

| Database Name | IN_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = an easement is present No = no easement is present Unknown = it is unknown if there is an easement present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

P_IN.39 – Inlet Consequence of Failure Rating

| Database Name | IN_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_IN.40 – Inlet Probability of Failure Rating

| Database Name | IN_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_IN.41 – Inlet Criticality to System

| Database Name | IN_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_IN.42 – Inlet Ownership Type

| Database Name | IN_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the basin |  |  |

P_IN.43 – Inlet Ownership Name

| Database Name | IN_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the basin |  |  |

P_IN.44 – Inlet Maintenance Authority Type

| Database Name | IN_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the structure |  |  |

P_IN.45 – Inlet Maintenance Authority Name

| Database Name | IN_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Scott County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the structure |  |  |

P_IN.46 – Inlet Holds Water

| Database Name | IN_HOLDS |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 10 | Domain | YesNoUnknown |
| Examples | Yes, No, Unknown |  |  |
| Description | Flag indicating if the structure holds water or not |  |  |

P_IN.47 – Inlet Infiltration Rate

| Database Name | IN_INFIL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value for infiltration rate [inches/hour]) |  |  |
| Description | Rate of infiltration through the bottom of an infiltration device in inches per hour |  |  |

P_IN.48 – Inlet Contributing Drainage Area

| Database Name | IN_CDA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in acres) |  |  |
| Description | Surface area that discharges to the structure, measured in acres |  |  |

P_IN.49 – Inlet Storage Volume

| Database Name | IN_STVL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in cubic feet) |  |  |
| Description | Storage volume of the inlet, in cubic feet |  |  |

P_IN.50 – Inlet Data Producer/Source Type

| Database Name | IN_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_IN.51 – Inlet Data Producer/Source Name

| Database Name | IN_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_IN.52 – Inlet Date Data Modified

| Database Name | IN_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the structure |  |  |

P_IN.53 – Inlet Data Source

| Database Name | IN_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the structure (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_IN.54 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory name where the structure is physically is located; if a structure centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the structure in; |  |  |

P_IN.55 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_IN.56 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the structure is located, please see links on page 25 for additional information; |  |  |

P_IN.57 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical structure is located |  |  |

P_IN.58 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_IN.59 – Inlet Comments

| Database Name | IN_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Conflicting records, unable to determine ownership of inlet" “Inlet shows extreme vertical cracking on west wall” “Centroid does not show correct position of inlet compared to as-builts” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of the basin feature |  |  |

Outlet Components

P_OUT.1 – Outlet Unique Identifier

| Database Name | OUT_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | 56210-65477 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner; Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_OUT.2 – Outlet Federated Identifier

| Database Name | OUT_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-56210-65477 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-56210-65477

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

56210-65477= Example of the locally-designated unique ID for the structure

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

Outlet ‘Flag’ Attributes: The following eight components (P_OUT.3 through P_OUT.10) enable the data creator to ‘flag’ each outlet with the types general features it contains. A single outlet may be flagged with one or more of the following characteristics; it was determined that this approach enables the stormwater data creator to have maximum flexibility for categorizing and attributing the features.

P_OUT.3 – Outlet Apron

| Database Name | OUT_APRON |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if an apron is present; |  |  |

P_OUT.4 – Outlet Outfall

| Database Name | OUT_OUTFL |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if the features is an outfall; |  |  |

>> What is the difference between an outlet and an outfall?

An outlet is any ending or discharge point in a system, whereas an outfall is the terminal end of a system where it discharges into a receiving water, or, it leaves one jurisdiction and enters another.

The definition provided by the Minnesota Stormwater Manual for an outfall is as follows:

“Outfall” means the point source where a municipal separate storm sewer system discharges to a receiving water, or the stormwater discharge permanently leaves the permittee’s municipal separate storm sewer system (a.k.a. MS4). It does not include diffuse runoff or conveyances that connect segments of the same stream or water systems (e.g., when a conveyance temporarily leaves an MS4 at a road crossing).

P_OUT.5 – Outlet Discharge Point

| Database Name | OUT_PDIS |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if the features is a discharge point |  |  |

P_OUT.6 – Outlet Ditch

| Database Name | OUT_DITCH |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if the features contains a ditch outlet |  |  |

P_OUT.7 – Outlet Underground

| Database Name | OUT_UNDER |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if the features is underground (subsurface); |  |  |

P_OUT.8 – Outlet Flapgate

| Database Name | OUT_FLGATE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if the feature has a flapgate. |  |  |

Photo at right illustrates an example of a flapgate at

an outlet >>

P_OUT.9 – Outlet Tide Chamber

| Database Name | OUT_TDCHM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag to indicate if the feature has a tide chamber |  |  |

This attribute (“Outlet Tide Chamber”) is likely to be removed from the next version of the schema, unless we receive enough confirmation from reviewer, partners, interests and agencies that it is needed or valuable to retain in the standard for their work.

P_OUT.10 – Outlet Type

| Database Name | OUT_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 100 | Domain | OutletTypeOptions |
| Examples | Pipe Culvert Weir v-notch Weir sharp crested Weir broad crested Weir proportional Weir other Riser structure single outlet Riser structure multiple outlet Drop Slotted Combination Other Unknown |  |  |
| Description | Domain of values to indicate the specific type of outlet; |  |  |

Contributions of additional values and ideas for improvement to this domain are strongly encouraged.

P_OUT.11 – Outlet Length

| Database Name | OUT_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Length in feet of the structure |  |  |

P_OUT.12 – Outlet Width

| Database Name | OUT_WID |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in inches) |  |  |
| Description | Width in inches of the structure |  |  |

P_OUT.13 – Outlet Height or Mean Depth

| Database Name | OUT_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Height or mean depth of the structure |  |  |

P_OUT.14 – Outlet Design Volume

| Database Name | OUT_VLD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of volume in cubic-feet) |  |  |
| Description | Volume of water in cubic feet the structure was designed to hold (if constructed) or holds naturally; |  |  |

P_OUT.15 – Outlet Invert Elevation

| Database Name | OUT_IELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value of invert elevation in feet above mean-sea-level) |  |  |
| Description | The elevation of the bottom of the inside portion of the outlet of the structure, in units of feet above mean sea level; |  |  |

P_OUT.16 – Outlet Rim Elevation

| Database Name | OUT_RELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Rim elevation (manholes); elevation of the center of the manhole lid measured from its top in feet above mean sea level |  |  |

P_OUT.17 – Outlet Bottom Elevation

| Database Name | OUT_BELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet above mean-sea-level) |  |  |
| Description | Bottom elevation of structure (differentiated from the invert elevation) |  |  |

P_OUT.18 – Outlet Inlet Rotation

| Database Name | OUT_ROTAT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (value = angle of rotation for cartographic symbol [azimuthal; north = 0°]) |  |  |

>> (Please see rotation example in Figure 5 on Page 59)

P_OUT.19 – Outlet Horizontal Position Accuracy Value

| Database Name | OUT_HPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_OUT.20) |  |  |
| Description | Indicator of the accuracy of x and y value of the structure |  |  |

P_OUT.20 – Outlet Horizontal Position Unit

| Database Name | OUT_HPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Outlet Horizontal Position Accuracy Value |  |  |

P_OUT.21 – Outlet Vertical Position Accuracy Value

| Database Name | OUT_VPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_OUT.22) |  |  |
| Description | Indicator of the accuracy of the z value of the structure |  |  |

P_OUT.22 – Outlet Vertical Position Unit

| Database Name | OUT_VPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Outlet Vertical Position Accuracy Value |  |  |

P_OUT.23 – Outlet Horizontal Datum

| Database Name | OUT_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_OUT.24 – Outlet Vertical Datum

| Database Name | OUT_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_OUT.25 – Outlet General Location

| Database Name | OUT_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | 2116 3nd Avenue SE SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 44.957459, -93.277684 44’ SE of intersection of 44th Street SW and Maine Avenue |  |  |
| Description | Data creator can provide general location information in the form of PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

P_OUT.26 – Outlet Drawing Link

| Database Name | OUT_ABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

P_OUT.27 – Outlet Document

| Database Name | OUT_ABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing of the structure |  |  |

P_OUT.28 – Outlet Structure Status

| Database Name | OUT_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the structure |  |  |

P_OUT.29– Outlet Status Date

| Database Name | OUT_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the status assessment of the structure |  |  |

P_OUT.30 – Outlet Condition

| Database Name | OUT_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition when inspected in spring 2018” “Damaged from flooding” |  |  |
| Description | A 150-character field for subjective written descriptions |  |  |

P_OUT.31 – Outlet Condition Date

| Database Name | OUT_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the condition description in P_OUT.30 – Outlet Condition |  |  |

P_OUT.32 – Outlet Installation Date

| Database Name | OUT_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the structure |  |  |

P_OUT.33 – Outlet Maintenance Agreement Number

| Database Name | OUT_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Document ID of agreement between agencies for the maintenance of the structure |  |  |

P_OUT.34 – Outlet Modification Date

| Database Name | OUT_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the structure |  |  |

P_OUT.35 – Outlet Easement

| Database Name | IN_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = an easement is present No = no easement is present Unknown = it is unknown if there is an easement present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

P_OUT.36 – Outlet Consequence of Failure Rating

| Database Name | OUT_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_OUT.37 – Outlet Probability of Failure Rating

| Database Name | OUT_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_OUT.38 – Outlet Criticality to System

| Database Name | OUT_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_OUT.39 – Outlet Ownership Type

| Database Name | OUT_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the basin |  |  |

P_OUT.40 – Outlet Ownership Name

| Database Name | OUT_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the basin |  |  |

P_OUT.41 – Outlet Maintenance Authority Type

| Database Name | OUT_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the structure |  |  |

P_OUT.42 – Outlet Maintenance Authority Name

| Database Name | OUT_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Scott County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the structure |  |  |

P_OUT.43 – Outlet Holds Water

| Database Name | OUT_HOLDS |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 10 | Domain | YesNoUnknown |
| Examples | Yes, No, Unknown |  |  |
| Description | Flag indicating if the structure holds water or not |  |  |

P_OUT.44 – Outlet Infiltration Rate

| Database Name | OUT_INFIL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value for infiltration rate [inches/hour]) |  |  |
| Description | Rate of infiltration through the bottom of an infiltration device in inches per hour; |  |  |

P_OUT.45 – Outlet Contributing Drainage Area

| Database Name | OUT_CDA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in acres) |  |  |
| Description | Surface area that discharges to the structure, measured in acres; |  |  |

P_OUT.46 – Outlet Storage Volume

| Database Name | OUT_STVL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in cubic feet) |  |  |
| Description | Storage volume of the inlet, in cubic feet |  |  |

P_OUT.47 – Outlet Data Producer/Source Type

| Database Name | OUT_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_OUT.48 – Outlet Data Producer/Source Name

| Database Name | OUT_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_OUT.49 – Outlet Date Data Modified

| Database Name | OUT_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the structure |  |  |

P_OUT.50 – Outlet Data Source

| Database Name | OUT_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the structure (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_OUT.51 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory where the structure is physically is located; if a structure centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the structure in |  |  |

P_OUT.52 –CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_OUT.53 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the structure is located, please see links on page 25 for additional information; |  |  |

P_OUT.54  – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical structure is located |  |  |

P_OUT.55 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_OUT.56 – Outlet Comments

| Database Name | OUT_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Conflicting records, unable to determine ownership of outlet” “Outlet damaged in most recent flood” “Data point does not show correct position of outlet compared to as-builts” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of structure; |  |  |

Manhole Components

P_MH.1 – Manhole Unique Identifier

| Database Name | MH_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | 2009-DRL226-RGC |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner; Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_MH.2 – Manhole Federated Identifier

| Database Name | MH_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-2009-DRL226-RGC |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-2009-DRL226-RGC

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

56210-65477= Example of the locally-designated unique ID for the structure

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

Manhole ‘Flag’ Attributes: The following eight components (P_MH.3 through P_MH.10) enable the data creator to ‘flag’ each Manhole with the types general features it contains. A single Manhole may be flagged with one or more of the following characteristics; it was determined that this approach enables the stormwater data creator to have maximum flexibility for categorizing and attributing the features.

P_MH.3 – Manhole Cleanout

| Database Name | MH_CO |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag if manhole is a cleanout |  |  |

P_MH.4 – Manhole Gate Valve

| Database Name | MH_GV |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag if manhole is a gate valve |  |  |

P_MH.5 – Manhole Junction Chamber

| Database Name | MH_JC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag if manhole is a junction chamber |  |  |

P_MH.6 – Manhole Control

| Database Name | MH_CN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag if manhole contains a control device |  |  |

P_MH.7 – Manhole Trap

| Database Name | MH_TR |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag if manhole is a trap |  |  |

P_MH.8 – Manhole Split

| Database Name | MH_SPLIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag if manhole is a split |  |  |

P_MH.9 – Manhole Sump

| Database Name | MH_SUMP |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag if manhole is a sump |  |  |

P_MH.10 – Manhole Buried

| Database Name | MH_BUR |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Flag if manhole is buried |  |  |

P_MH.11 – Manhole Length

| Database Name | MH_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Length in feet of the structure |  |  |

P_MH.12 – Manhole Width

| Database Name | MH_WID |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in inches) |  |  |
| Description | Width in inches of the structure |  |  |

P_MH.13 – Manhole Height or Mean Depth

| Database Name | MH_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Height or mean depth of the structure |  |  |

P_MH.14 – Manhole Design Volume

| Database Name | MH_VLD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in cubic-feet) |  |  |
| Description | Volume of water in cubic feet the structure was designed to hold (if constructed) or holds naturally; |  |  |

P_MH.15 – Manhole Invert Elevation

| Database Name | MH_IELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet above mean-sea-level) |  |  |
| Description | The elevation of the bottom of the inside portion of the Manhole of the structure, in units of feet above mean sea level; |  |  |

P_MH.16 – Manhole Rim Elevation

| Database Name | MH_RELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet above mean-sea-level) |  |  |
| Description | Rim elevation (manholes); elevation of the center of the manhole lid measured from its top in feet above mean sea level |  |  |

P_MH.17 – Manhole Bottom Elevation

| Database Name | MH_BELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Bottom elevation of structure (differentiated from the invert elevation) |  |  |

P_MH.18 – Manhole Inlet Rotation

| Database Name | MH_ROTAT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (value = angle of rotation for cartographic symbol [azimuthal; north = 0°]) |  |  |
| Description | Recommended angle of rotation for cartographic symbol; this field is used to provide a value for the preferred rotation of the point symbol for proper mapping display; the 360° azimuthal system is used (clockwise rotation) (e.g.: north= 0°, east = 90°, south= 180°, west = 270°) |  |  |

>> (Please see rotation example in Figure 5 on Page 59)

P_MH.19 – Manhole Horizontal Position Accuracy Value

| Database Name | MH_HPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_MH.20) |  |  |
| Description | Indicator of the accuracy of x and y value of the structure |  |  |

P_MH.20 – Manhole Horizontal Position Unit

| Database Name | MH_HPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Manhole Horizontal Position Accuracy Value |  |  |

P_MH.21 – Manhole Vertical Position Accuracy Value

| Database Name | MH_VPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_MH.22) |  |  |
| Description | Indicator of the accuracy of the z value of the structure |  |  |

P_MH.22 – Manhole Vertical Position Unit

| Database Name | MH_VPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Manhole Vertical Position Accuracy Value |  |  |

P_MH.23 – Manhole Vertical Datum

| Database Name | MH_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_MH.24 – Manhole Horizontal Datum

| Database Name | MH_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_MH.25 – Manhole General Location

| Database Name | MH_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | 2116 3nd Avenue SE SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 44.957459, -93.277684 44’ SE of intersection of 44th Street SW and Maine Avenue |  |  |
| Description | Data creator can provide general location information in the form of PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

P_MH.26 – Manhole Drawing Link

| Database Name | MH_ABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

P_MH.27 – Manhole Document

| Database Name | MH_ABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing of the structure |  |  |

P_MH.28 – Manhole Structure Status

| Database Name | MH_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the structure |  |  |

P_MH.29 – Manhole Status Date

| Database Name | MH_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the status assessment of the structure |  |  |

P_MH.30 – Manhole Condition

| Database Name | MH_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition when inspected in spring 2018” “Damaged from flooding” |  |  |
| Description | A 150-character field for subjective written descriptions |  |  |

P_MH.31 – Manhole Condition Date

| Database Name | MH_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the condition description in P_MH.30 – Manhole Condition |  |  |

P_MH.32 – Manhole Installation Date

| Database Name | MH_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the structure |  |  |

P_MH.33 – Manhole Maintenance Agreement Number

| Database Name | MH_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Document ID of agreement between agencies for the maintenance of the structure |  |  |

P_MH.34 – Manhole Modification Date

| Database Name | MH_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the structure |  |  |

P_MH.35 – Manhole Easement

| Database Name | MH_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = an easement is present No = no easement is present Unknown = it is unknown if there is an easement present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

P_MH.36 – Manhole Consequence of Failure Rating

| Database Name | MH_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_MH.37 – Manhole Probability of Failure Rating

| Database Name | MH_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_MH.38 – Manhole Criticality to System

| Database Name | MH_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_MH.39 – Manhole Ownership Type

| Database Name | MH_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the manhole/structure |  |  |

P_MH.40 – Manhole Ownership Name

| Database Name | MH_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the manhole/structure |  |  |

P_MH.41 – Manhole Maintenance Authority Type

| Database Name | MH_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the structure |  |  |

P_MH.42 – Manhole Maintenance Authority Name

| Database Name | MH_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Scott County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the structure |  |  |

P_MH.43 – Manhole Data Producer/Source Type

| Database Name | MH_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_MH.44 – Manhole Data Producer/Source Name

| Database Name | MH_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_MH.45 – Manhole Date Data Modified

| Database Name | MH_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the structure |  |  |

P_MH.46– Manhole Data Source

| Database Name | MH_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the structure (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_MH.47 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory where the structure is physically is located; if a structure centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the structure in; |  |  |

P_MH.48 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_MH.49 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the structure is located, please see links on page 25 for additional information; |  |  |

P_MH.50  – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical structure is located |  |  |

P_MH.51 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_MH.52 – Manhole Comments

| Database Name | MH_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Manhole under 6” of asphalt, excavated on June 12, 2017” “Manhole damaged in flooding event August 2019” “Data point does not show correct position of manhole compared to as-builts” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of structure; |  |  |

Lift Station Components

P_LS.1 – Lift Station Unique Identifier

| Database Name | LS_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | LS-516-013 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner; Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_LS.2 – Lift Station Federated Identifier

| Database Name | LS_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-LS-516-013 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-LS-516-013

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

LS-516-013 = Example of the locally-designated unique ID for the structure

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

P_LS.3 – Lift Station Type

| Database Name | LS_Type |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | Mandatory |
| Width | Default | Domain | LiftStationType |
| Example | Dry Submersible Simplex pump Duplex pump Triplex pump Jockey pump Turbine pump Split-case lift Wet-dry configuration Centrifugal pump Other Unknown |  |  |
| Description | Total number of wells at the lift station |  |  |

Wet-Dry Lift Station                                                                    Lift Station with Duplex Pumps

P_LS.4 – Lift Station Number of Wells

| Database Name | LS_NOW |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert number of wells |  |  |
| Description | Total number of wells at the lift station |  |  |

P_LS.5 – Lift Station Number of Pumps

| Database Name | LS_NOP |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert number of pumps |  |  |
| Description | Total number of pumps at the lift station |  |  |

P_LS.6 – Lift Station SCADA System Information

| Database Name | LS_SCADA |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 100 | Domain | (none) |
| Example | (Description of SCADA information) |  |  |
| Description | Relevant descriptive information about the attendant SCADA system in use at the site |  |  |

P_LS.7 – Lift Station Maximum Discharge Capacity

| Database Name | LS_MXDCAP |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example |  |  |  |
| Description | Relevant descriptive information about the attendant SCADA system in use at the site |  |  |

P_LS.8 – Foundation Drain

| Database Name | LS_FD |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Indicates the presence of a foundation drain |  |  |

P_LS.9 – Lift Station Sump

| Database Name | LS_SUMP |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 7 | Domain | YesNoUnknown |
| Example | Yes, No, Unknown |  |  |
| Description | Indicates the presence of a sump |  |  |

P_LS.10 – Lift Station Length

| Database Name | LS_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Length in feet of the structure |  |  |

P_LS.11 – Lift Station Width

| Database Name | LS_WID |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in inches) |  |  |
| Description | Width in inches of the structure |  |  |

P_LS.12 – Lift Station Height

| Database Name | LS_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Height of the structure |  |  |

P_LS.13 – Lift Station Depth

| Database Name | LS_DEP |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Depth of the structure |  |  |

P_LS.14 – Lift Station Design Volume

| Database Name | LS_VLD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in cubic-feet) |  |  |
| Description | Volume of water in cubic feet the structure was designed to hold; |  |  |

P_LS.15 – Lift Station Invert Elevation

| Database Name | LS_IELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet above mean-sea-level) |  |  |
| Description | The elevation of the invert of the Lift Station of the structure, in units of feet above mean sea level; |  |  |

P_LS.16 – Lift Station Outlet Elevation

| Database Name | LS_OELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet above mean-sea-level) |  |  |
| Description | The elevation of the bottom of the inside portion of the Lift Station of the structure, in units of feet above mean sea level; |  |  |

P_LS.17 – Lift Station Rim Elevation

| Database Name | LS_RELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Rim elevation (Lift Stations); elevation of the center of the Lift Station lid measured from its top in feet above mean sea level |  |  |

P_LS.18 – Lift Station Bottom Elevation

| Database Name | LS_BELEV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value feet above mean-sea-level) |  |  |
| Description | Bottom elevation of structure (differentiated from the invert elevation) |  |  |

P_LS.19 – Lift Station Inlet Rotation

| Database Name | LS_ROTAT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (value = angle of rotation for cartographic symbol [azimuthal; north = 0°]) |  |  |
| Description | Recommended angle of rotation for cartographic symbol; this field is used to provide a value for the preferred rotation of the point symbol for proper mapping display; the 360° azimuthal system is used (clockwise rotation) (e.g.: north= 0°, east = 90°, south= 180°, west = 270°) |  |  |

>> (Please see rotation example in Figure 5 on Page 59)

P_LS.20 – Lift Station Horizontal Position Accuracy Value

| Database Name | LS_HPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_LS.21) |  |  |
| Description | Indicator of the accuracy of x and y value of the structure |  |  |

P_LS.21 – Lift Station Horizontal Position Unit

| Database Name | LS_HPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Lift Station Horizontal Position Accuracy Value |  |  |

P_LS.22 – Lift Station Vertical Position Accuracy Value

| Database Name | LS_VPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_LS.23) |  |  |
| Description | Indicator of the accuracy of the z value of the structure |  |  |

P_LS.23 – Lift Station Vertical Position Unit

| Database Name | LS_VPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Lift Station Vertical Position Accuracy Value |  |  |

P_LS.24 – Lift Station Vertical Datum

| Database Name | LS_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_LS.25 – Lift Station Horizontal Datum

| Database Name | LS_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_LS.26 – Lift Station General Location

| Database Name | LS_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | 2116 3nd Avenue SE SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 44.957459, -93.277684 44’ SE of intersection of 44th Street SW and Maine Avenue |  |  |
| Description | Data creator can provide general location information in the form of PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

P_LS.27 – Lift Station Drawing Link

| Database Name | LS_ABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

P_LS.28 – Lift Station Document

| Database Name | LS_ABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing of the structure |  |  |

P_LS.29 – Lift Station Structure Status

| Database Name | LS_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the structure |  |  |

P_LS.30 – Lift Station Status Date

| Database Name | LS_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the status assessment of the structure |  |  |

P_LS.31 – Lift Station Condition

| Database Name | LS_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition when inspected in spring 2018” “Damaged from flooding” |  |  |
| Description | A 150-character field for subjective written descriptions |  |  |

P_LS.32 – Lift Station Condition Date

| Database Name | LS_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the condition description in P_LS.31 – Lift Station Condition |  |  |

P_LS.33 – Lift Station Installation Date

| Database Name | LS_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the structure |  |  |

P_LS.34 – Lift Station Maintenance Agreement Number

| Database Name | LS_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Document ID of agreement between agencies for the maintenance of the structure |  |  |

P_LS.35 – Lift Station Modification Date

| Database Name | LS_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the structure |  |  |

P_LS.36 – Lift Station Easement

| Database Name | LS_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = an easement is present No = no easement is present Unknown = it is unknown if there is an easement present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

P_LS.37 – Lift Station Consequence of Failure Rating

| Database Name | LS_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_LS.38 – Lift Station Probability of Failure Rating

| Database Name | LS_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_LS.39 – Lift Station Criticality to System

| Database Name | LS_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_LS.40 – Lift Station Ownership Type

| Database Name | LS_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the lift station/structure |  |  |

P_LS.41 – Lift Station Ownership Name

| Database Name | LS_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the lift station/structure |  |  |

P_LS.42 – Lift Station Maintenance Authority Type

| Database Name | LS_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the structure |  |  |

P_LS.43 – Lift Station Maintenance Authority Name

| Database Name | LS_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Scott County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the structure |  |  |

P_LS.44 – Lift Station Holds Water

| Database Name | LS_HOLDS |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 10 | Domain | YesNoUnknown |
| Examples | Yes, No, Unknown |  |  |
| Description | Flag indicating if the structure holds water or not |  |  |

P_LS.45 – Lift Station Contributing Drainage Area

| Database Name | LS_CDA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in acres) |  |  |
| Description | Surface area that discharges to the structure, measured in acres; |  |  |

P_LS.46 – Lift Station Storage Volume

| Database Name | LS_STVL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in cubic feet) |  |  |
| Description | Storage volume of the lift station/structure, in cubic feet |  |  |

P_LS.47 – Lift Station Data Producer/Source Type

| Database Name | LS_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_LS.48 – Lift Station Data Producer/Source Name

| Database Name | LS_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_LS.49 – Lift Station Date Data Modified

| Database Name | LS_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the structure |  |  |

P_LS.50– Lift Station Data Source

| Database Name | LS_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the structure (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_LS.51 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territorywhere the structure is physically is located; if a structure centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the structure in; |  |  |

P_LS.52 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_LS.53 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the structure is located, please see links on page 25 for additional information; |  |  |

P_LS.54  – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical structure is located |  |  |

P_LS.55 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_LS.56 – Lift Station Comments

| Database Name | LS_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “Lift Station damaged in flooding event of August 2019” “Data point does not show correct position of Lift Station compared to as-built drawings on file” “Lift station was built on the property line” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of structure; |  |  |

Best Management Practices (BMP) Components

P_BMP.1 – BMP Unique Identifier

| Database Name | BMP_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | 09021970-616 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner; Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_BMP.2 – BMP Federated Identifier

| Database Name | BMP_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202-09021970-616 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-09021970-616

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

LS-516-013 = Example of the locally-designated unique ID for the structure

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

P_BMP.3 – BMP Type

| Database Name | BMP_Type |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 45 | Domain | BMPType |
| Examples | Amended-composted soils, bioretention-rain garden, ditch block, dry pond, filtration basin (no underdrain), filtration basin (with underdrain), filtration bench/shelf (no underdrain), filtration bench/shelf (with underdrain), filtration swale (no underdrain), filtration swale/shelf (with underdrain), green roof, iron enhanced filter, infiltration trench, infiltration basin, sand filter, stormwater pond/wet pond, tree box, offline-basin, permeable pavement, planter, porous pavers, porous concrete, etc. (for complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |
| Description | Indication of the BMP type |  |  |

One of the challenges of developing a multi-purpose stormwater geodata standard is how to best capture complex fixtures within the context of points, lines and polygons. The current version of the standard represents BMP fixtures as points, however, there may be some features which are better suited for representation as linear features. We encourage stakeholder comments on how to best advance this discussion toward a solution.

Examples of stormwater BMPs

Infiltration basin			                     Infiltration trench (with mulch)

Filtration with underdrain				      Filtration pond with iron-enhanced sand

(with vegetation-high maintenance)

Filtration with underdrain				        Ditch blocks in channel

(with vegetation-high maintenance)			      (w/ vegetation-low maintenance)

P_BMP.4 – BMP Length

| Database Name | BMP_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Length in feet of the structure |  |  |

P_BMP.5 – BMP Width

| Database Name | BMP_WID |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in inches) |  |  |
| Description | Width in inches of the structure |  |  |

P_BMP.6 – BMP Height or Mean Depth

| Database Name | BMP_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Height or mean depth of the structure |  |  |

P_BMP.7 – BMP Elevation

| Database Name | BMP_ELV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet above mean sea level) |  |  |
| Description | Elevation of structure in mean feet above sea level |  |  |

P_BMP.8 – BMP Surface Area

| Database Name | BMP_SAREA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in square feet) |  |  |
| Description | Surface area of the structure, fixture or features in square feet |  |  |

P_BMP.9 – BMP Rotation

| Database Name | BMP_ROTAT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Value for preferred rotation angle for display of cartographic feature) |  |  |
| Description | Recommended angle of rotation for cartographic symbol; this field is used to provide a value for the preferred rotation of the point symbol for proper mapping display; the 360° azimuthal system is used (clockwise rotation) (e.g.: north= 0°, east = 90°, south= 180°, west = 270°) |  |  |

>> (Please see rotation example in Figure 5 on Page 59)

P_BMP.10 – BMP Design Volume

| Database Name | BMP_VLD |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in cubic-feet) |  |  |
| Description | Volume of water in cubic feet the structure was designed to hold; |  |  |

The current version of the MSWGP v. 0.5 draft standard does not presently include the following listed attributes for BMPs, however, these can be included in the next version if they are deemed valuable and needed by the stakeholder community after their review and comment period:

- Invert Elevation

- Outlet Elevation

- Rim Elevation

- Bottom Elevation

The MSWGP Steering Team encourages and welcomes stakeholder feedback, ideas, suggestions and input on how to best represent the wide range of BMP features consistently and effectively in GIS.

P_BMP.11 – BMP Horizontal Position Accuracy Value

| Database Name | BMP_HPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_BMP.12) |  |  |
| Description | Indicator of the accuracy of x and y value of the structure |  |  |

P_BMP.12 – BMP Horizontal Position Unit

| Database Name | BMP_HPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the BMP Horizontal Position Accuracy Value |  |  |

P_BMP.13 – BMP Vertical Position Accuracy Value

| Database Name | BMP_VPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_BMP.14) |  |  |
| Description | Indicator of the accuracy of the z value of the structure |  |  |

P_BMP.14 – BMP Vertical Position Unit

| Database Name | BMP_VPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the BMP Vertical Position Accuracy Value |  |  |

P_BMP.15 – BMP Vertical Datum

| Database Name | BMP_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_BMP.16 – BMP Horizontal Datum

| Database Name | BMP_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_BMP.17 – BMP General Location

| Database Name | BMP_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | 2116 3nd Avenue SE SW ¼ of the NE ¼ of the NE ¼ of T29N R24E S12 44.957459, -93.277684 32’ SE of intersection of 34th Street SW and Maine Avenue |  |  |
| Description | Data creator can provide general location information in the form of PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

P_BMP.18 – BMP Filter Material

| Database Name | BMP_FM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 30 | Domain | FilterMaterial |
| Examples | Gravel, Biochar, Amended Soil, Rock, Sand-Compost Mix, Sand Filter, Iron-Enhanced Filter, Soil, Spent Lime, Other, Unknown |  |  |
| Description | Filter material type |  |  |

P_BMP.19 – BMP Ground Cover

| Database Name | BMP_GC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 30 | Domain | GroundCover |
| Examples | Asphalt, Brick, Concrete, Gravel, Mulch, Bare Soil, Vegetation-high maintenance, Vegetation-low maintenance, Vegetation-native, Other, Unknown |  |  |
| Description | Ground cover type |  |  |

P_BMP.20 – BMP Drawing Link

| Database Name | BMP_ABLINK |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert link/URL accessing as-built drawing) |  |  |
| Description | URL/weblink to the as-built drawing containing the structure |  |  |

P_BMP.21 – BMP Document

| Database Name | BMP_ABDOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 150 | Domain | (no domain) |
| Example | (insert document number, ID number, reference number of as-built drawing) |  |  |
| Description | Document number, ID number, or reference number of the original as-built drawing of the structure |  |  |

P_BMP.22 – BMP Structure Status

| Database Name | BMP_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 20 | Domain | Status |
| Examples | Active, Inactive, Failed, Removed, Proposed, Abandoned, Under Construction, Other, Unknown |  |  |
| Description | Status indicator of the structure |  |  |

P_BMP.23 – BMP Status Date

| Database Name | BMP_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the status assessment of the structure |  |  |

P_BMP.24 – BMP Condition

| Database Name | BMP_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “Appeared in good condition when inspected in spring 2018” “Damaged from flooding” |  |  |
| Description | A 150-character field for subjective written descriptions |  |  |

P_BMP.25 – BMP Condition Date

| Database Name | BMP_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the condition description in P_BMP.24 – BMP Condition |  |  |

P_BMP.26 – BMP Installation Date

| Database Name | BMP_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of installation of the structure |  |  |

P_BMP.27 – BMP Maintenance Agreement Number

| Database Name | BMP_MAGRN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Example | (insert example of Maintenance Agreement Number/ID) |  |  |
| Description | Document ID of agreement between agencies for the maintenance of the structure |  |  |

P_BMP.28 – BMP Modification Date

| Database Name | BMP_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 2/14/2020 |  |  |
| Description | Date of the last modification of the structure |  |  |

P_BMP.29 – BMP Easement

| Database Name | BMP_EASM |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 7 | Domain | YesNoUnknown |
| Examples | Yes = an easement is present No = no easement is present Unknown = it is unknown if there is an easement present |  |  |
| Description | Flag to indicate if there is an easement present |  |  |

P_BMP.30 – BMP Consequence of Failure Rating

| Database Name | BMP_COF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of consequence of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_BMP.31 – BMP Probability of Failure Rating

| Database Name | BMP_POF |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of severity of probability of failure of asset 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_BMP.32 – BMP Criticality to System

| Database Name | BMP_CRIT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 1 | Domain | CriticalRating |
| Examples | (see values in Description below) |  |  |
| Description | Rating: 1-5 of criticality of the asset (1 = low, 5=high) 1 = Low 2 = Medium Low 3 = Medium 4 = Medium High 5 = High |  |  |

P_BMP.33 – BMP Ownership Type

| Database Name | BMP_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the structure |  |  |

P_BMP.34 – BMP Ownership Name

| Database Name | BMP_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the structure |  |  |

P_BMP.35 – BMP Maintenance Authority Type

| Database Name | BMP_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the structure |  |  |

P_BMP.36 – BMP Maintenance Authority Name

| Database Name | BMP_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Scott County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the structure |  |  |

P_BMP.37 – BMP Holds Water

| Database Name | BMP_HOLDS |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 10 | Domain | YesNoUnknown |
| Examples | Yes, No, Unknown |  |  |
| Description | Flag indicating if the structure holds water |  |  |

P_BMP.38 – BMP Infiltration Rate

| Database Name | BMP_INF |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in inches per hour) |  |  |
| Description | Numerical indicator of infiltration through the bottom of the BMP; |  |  |

P_BMP.39 – BMP Designed Treatment Volume

| Database Name | BMP_DTV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in cubic-feet per hour) |  |  |
| Description | Volume of water the BMP is designed to treat in cubic fee per hour |  |  |

P_BMP.40 – BMP Storage Volume

| Database Name | BMP_STVL |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in cubic feet) |  |  |
| Description | Storage volume of the structure, in cubic feet |  |  |

P_BMP.41 – Lift Station Contributing Drainage Area

| Database Name | BMP_CDA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in acres) |  |  |
| Description | Surface area that discharges to the structure, measured in acres |  |  |

P_BMP.42 – BMP Data Producer/Source Type

| Database Name | BMP_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_BMP.43 – BMP Data Producer/Source Name

| Database Name | BMP_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_BMP.44 – BMP Date Data Modified

| Database Name | BMP_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the structure |  |  |

P_BMP.45– BMP Data Source

| Database Name | BMP_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the structure (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_BMP.46 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory where the structure is physically is located; if a structure centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the structure in |  |  |

P_BMP.47 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_BMP.48 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the structure is located, please see links on page 25 for additional information |  |  |

P_BMP.49 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the physical structure is located |  |  |

P_BMP.50 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_BMP.51 – BMP Comments

| Database Name | BMP_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | “BMP damaged in flooding event August 2019” “Data point does not show correct position of BMP compared to as-built drawings on file” “BMP was built on the property line” |  |  |
| Description | General field for text comments related to either the physical or digital aspects of structure |  |  |

Monitoring Components

P_MON.1 – Monitor Unique Identifier

| Database Name | MON_ORID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Example | MON501-055 |  |  |
| Description | Original unique identifier provided by the original source or data provider; Primary key for the asset as used by the asset owner; Locally-designated ID, containing any combination of letter, hyphens or numbers as needed by the data producer; |  |  |

P_MON.2 – Monitor Federated Identifier

| Database Name | MON_FID |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 90 | Domain | (no domain) |
| Example | 2705300664202- MON501-055 |  |  |
| Description | Original unique identifier provided by the original source or data provider with prefix appended to it indicating state, county and municipal code; |  |  |

The purpose of the ‘federated ID’ is to enable the creation of a unique ID which concatenates the original local ID to a set of codes which indicate the jurisdiction in which the pipe is found.

In the example above we have a federated ID of:

2705300664202-MON501-055

Where:

27 = FIPS/ANSI Code for Minnesota

053 = FIPS/ANSI Code for Hennepin County

00664202 = CTU Code for Fort Snelling Unorganized Territory

LS-516-013 = Example of the locally-designated unique ID for the structure

The FIPS and CTU codes are maintained in the General Elements of the feature.

Please note: These number codes are already used extensively in other standards already adopted by the Minnesota Geospatial Advisory Council.

P_MON.3 – Monitor Category

| Database Name | MON_CAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 45 | Domain | MonitorCategory |
| Examples | Sensor Monitoring Well Sampler Gauge Other Unknown |  |  |
| Description | Category of monitoring device placed, used or installed |  |  |

P_MON.4 – Monitor Type

| Database Name | MON_TYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 45 | Domain | MonitorType |
| Examples | Frost sensor Pressure transducer Water level sensor Temperature sensor Conductivity sensor Sonic distance sensor Laser velocity sensor Automated sampler Staff gauge Rain gauge Bubbler for water depth Turbidity sensor pH sensor Dissolved oxygen sensor Algae sensor Chlorophyll sensor Soil moisture gauge Monitoring well, water level only Monitoring well, temporary dewatering, construction Monitoring well, remedial site investigation, water quality Monitoring well, geological, exploratory Monitoring well, general water quality Multi-parameter sensor Frost sensor Other Unknown |  |  |
| Description | Indication of the specific monitor type |  |  |

P_MON.5 – Monitor Length

| Database Name | MON_LNG |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Length in feet of the monitoring component |  |  |

P_MON.6 – Monitor Width

| Database Name | MON_WID |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in inches) |  |  |
| Description | Width in inches of the monitoring component |  |  |

P_MON.7 – Monitor Height or Mean Depth

| Database Name | MON_HT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet) |  |  |
| Description | Height or mean depth of the structure |  |  |

P_MON.8 – Monitor Elevation

| Database Name | MON_ELV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Insert value in feet above mean sea level) |  |  |
| Description | Elevation of monitoring component in mean feet above sea level |  |  |

P_MON.9 – Monitor Rotation

| Database Name | MON_ROTAT |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | (Value for preferred rotation angle for display of cartographic feature) |  |  |
| Description | Recommended angle of rotation for cartographic symbol; this field is used to provide a value for the preferred rotation of the point symbol for proper mapping display; the 360° azimuthal system is used (clockwise rotation) (e.g.: north= 0°, east = 90°, south= 180°, west = 270°) |  |  |

>> (Please see rotation example in Figure 5 on Page 59)

P_MON.10 – Monitor Horizontal Position Accuracy Value

| Database Name | MON_HPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_MON.11) |  |  |
| Description | Indicator of the accuracy of x and y value of the structure |  |  |

P_MON.11 – Monitor Horizontal Position Unit

| Database Name | MON_HPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the Monitor Horizontal Position Accuracy Value |  |  |

P_MON.12 – Monitor Vertical Position Accuracy Value

| Database Name | MON_VPAV |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (none) |
| Example | Insert numerical value of the positional accuracy (See P_MON.13) |  |  |
| Description | Indicator of the accuracy of the z value of the structure |  |  |

P_MON.13 – Monitor Vertical Position Unit

| Database Name | MON_VPU |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 8 | Domain | AccuracyMeasure |
| Examples | Feet Meters |  |  |
| Description | Indicates unit of measurement (in either feet or meters) for the MON Vertical Position Accuracy Value |  |  |

P_MON.14 – Monitor Vertical Datum

| Database Name | MON_VDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | VDatum |
| Examples | Ellipsoidal, NAD83 (HARN) Orthometric, NAVD88 From as-built drawing Unknown datum |  |  |
| Description | Name of the vertical datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_MON.15 – Monitor Horizontal Datum

| Database Name | MON_HDAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 50 | Domain | HDatum |
| Examples | RTCM_23_NAD83(2011) CMRx_NAD83(1996) From as-built drawing Unknown datum |  |  |
| Description | Name of the horizontal datum in use by the data producer in creating their data or taken from as-built drawings (For complete list of draft domain values, please see the file SGTS_V_0_5_Domains.xlsx) |  |  |

P_MON.16 – Monitor General Location

| Database Name | MON_LOC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 100 | Domain | (no domain) |
| Examples | South end of Bear Lake Appx. 15’ downstream from Xcel Energy outfall on Black Dog Lake 46.546947, -94.281779 Narrow channel between Lower Cullen Lake and Middle Cullen Lake |  |  |
| Description | Data creator can provide general location information in the form of PLSS description, latitude/longitude coordinates, address, intersection or other descriptive location |  |  |

P_MON.17 – Monitor Brand

| Database Name | MON_BRAND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 50 | Domain | (no domain) |
| Examples | (Insert brand name of monitoring device) |  |  |
| Description | Brand of monitoring device |  |  |

P_MON.18 – Monitor Device Maintenance

| Database Name | MON_DMAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | (no domain) |
| Examples | (Insert numerical value and unit type? Example: 3 months?) |  |  |
| Description | Period of time between recommended maintenance activities |  |  |

P_MON.19 – Monitor Device Maintenance

| Database Name | MON_MNTYPE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 120 | Domain | (no domain) |
| Examples | (Insert maintenance type of monitoring device) |  |  |
| Description | Type of maintenance to be performed |  |  |

P_MON.20 – Monitor Manufacturer Date

| Database Name | MON_MFDT |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of manufacture of the monitoring device |  |  |

P_MON.21 – Monitor Status

| Database Name | MON_STAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 18 | Domain | MonitorStatus |
| Examples | Installed, Active-temporary, Active-continuous, Inactive-monitor in place Other, Unknown |  |  |
| Description | Status of monitoring device |  |  |

P_MON.22 – Monitor Status Date

| Database Name | MON_SDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last condition assessment of the monitoring device |  |  |

P_MON.23 – Monitor Installation Date

| Database Name | MON_IDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of installation of the monitoring device |  |  |

P_MON.24 – Monitor Modification Date

| Database Name | MON_MDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of most recent modification of the monitoring device |  |  |

P_MON.25 – Monitor Expiration Date

| Database Name | MON_XDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of expiration of the monitoring device |  |  |

P_MON.26 – Monitor Condition

| Database Name | MON_COND |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Optional |
| Width | 150 | Domain | (no domain) |
| Example | “As of July 2019, monitor is in place and functioning well” |  |  |
| Description | A150-character field for subjective written descriptions about the monitoring device |  |  |

P_MON.27 – Monitor Condition Date

| Database Name | MON_CDATE |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Example | 7/31/2020 |  |  |
| Description | Date of the condition description in P_MON.26 – MON Condition |  |  |

P_MON.28 – Monitor Ownership Type

| Database Name | MON_OWNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which owns the monitoring device |  |  |

P_MON.29 – Monitor Ownership Name

| Database Name | MON_OWNN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Ramsey County State of Minnesota |  |  |
| Description | Name of the entity or agency which owns the monitoring device |  |  |

P_MON.30 – Monitor Maintenance Authority Type

| Database Name | MON_MAINT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which maintains the monitoring device |  |  |

P_MON.31 – Monitor Maintenance Authority Name

| Database Name | MON_MAINN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | If Available |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Scott County State of Minnesota |  |  |
| Description | Name of the entity or agency which maintains the monitoring device |  |  |

P_MON.32 – Monitor Contributing Drainage Area

| Database Name | MON_CDA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | If Available |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in acres) |  |  |
| Description | Size of drainage area that discharges to the monitor, measured in acres; |  |  |

P_MON.33 – Monitor Data Producer/Source Type

| Database Name | MON_DATAT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | ManagementType |
| Examples | Township, City, County, Judicial, State, Regional Agency, State, Federal, Watershed Management Unit, Educational Entity, Private, Other, Unknown |  |  |
| Description | Type of entity or agency which produces or is the source of the data |  |  |

P_MON.34 – Monitor Data Producer/Source Name

| Database Name | MON_DATAN |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Conditional |
| Width | 75 | Domain | AgencyOwnMaintain |
| Examples | Buffalo-Red River Watershed District City of Eagan City of Bloomington Metropolitan Council Hennepin County State of Minnesota |  |  |
| Description | Name of entity or agency which produces or is the source of the data |  |  |

P_MON.35 – Monitor Date Data Modified

| Database Name | MON_DAMOD |  |  |
| --- | --- | --- | --- |
| Data Type | Date | Inclusion | If Available |
| Width | Default | Domain | (no domain) |
| Examples | 2/14/2020 |  |  |
| Description | Date of last modification to the digital feature representing the monitor |  |  |

P_MON.36 – Monitor Data Source

| Database Name | MON_DASRC |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | (no domain) |
| Examples | A. Blenkush, Hennepin County GIS Staff C. Magnuson, RWMWD Ramsey County GIS Department Anoka County Public Works |  |  |
| Description | Name of source, providing agency, interest or company of the digital data representing the structure (Note: provider of the digital data may differ from owner and/or the maintainer of the physical asset); This can be an individual, department, agency, etc. |  |  |

P_MON.37 – CTU Name

| Database Name | CTU_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 75 | Domain | CTU_Name |
| Examples | Eagan Bloomington |  |  |
| Description | Name of the city, township or unorganized territory where the monitor is physically is located; if a structure centroid point is directly on a boundary between two cities/counties, the data creator may use their discretion as to which municipality or county they place the structure in; |  |  |

P_MON.38 – CTU Code

| Database Name | CTU_ID_TXT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 8 | Domain | CTUIDText |
| Examples | 02394586 = City of Eagan 02394198 = City of Bloomington |  |  |
| Description | Eight-digit CTU code representing the municipal unit (city, township, unorganized territory) where the asset is located; as the leadings zeros are needed and this is not to be used for any mathematical uses, this ID field is a text field (TXT), please see the links on page 25 of this document for additional context |  |  |

P_MON.39 – County Code

| Database Name | CO_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 3 | Domain | CountyCode |
| Examples | 037 = Dakota County 053 = Hennepin County |  |  |
| Description | Three-digit (FIPS/ANSI) code representing the county where the monitor is located, please see links on page 25 for additional information; |  |  |

P_MON.40 – County Name

| Database Name | CO_NAME |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 40 | Domain | CountyName |
| Examples | Dakota Hennepin |  |  |
| Description | Name of the county where the monitor is located |  |  |

P_MON.41 – State Code

| Database Name | STATE_CODE |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 2 | Domain | (no domain) |
| Example | 27 |  |  |
| Description | There is only one value for Minnesota 27 = FIPS/ANSI Code for Minnesota |  |  |

P_MON.42 – Monitor Comments

| Database Name | MON_CMNT |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 254 | Domain | (no domain) |
| Examples | (insert comment text) |  |  |
| Description | General field for text comments related to either the physical or digital aspects of monitor; |  |  |

Basins (As Polygon Features)

P_POLY.1 – (Basin) Polygon Method

| Database Name | POLY_METH |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 24 | Domain | PolyMethod |
| Example | Digitized GPS point collection LIDAR extract Imagery extract Import from CAD External GIS source Other Unknown |  |  |
| Description | Method for the creation of the polygon representing the basin feature |  |  |

P_POLY.2 – (Basin) Polygon Perimeter

| Database Name | POLY_PERI |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | Mandatory |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in feet of perimeter of the polygon) |  |  |
| Description | Value in feet of the perimeter of the polygon representing the basin; Can be calculated within GIS; |  |  |

P_POLY.3 – (Basin) Polygon Surface Area

| Database Name | POLY_SAREA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | Mandatory |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in feet of perimeter of the polygon) |  |  |
| Description | Area value (in acres) of the polygon representing the basin; Can be calculated within GIS; |  |  |

Best Management Practices - BMPs (As Polygon Features)

P_POLY.1 – (BMP) Polygon Method

| Database Name | POLY_METH |  |  |
| --- | --- | --- | --- |
| Data Type | Text | Inclusion | Mandatory |
| Width | 24 | Domain | PolyMethod |
| Example | Digitized GPS point collection LIDAR extract Imagery extract Import from CAD External GIS source Other Unknown |  |  |
| Description | Method for the creation of the polygon representing the BMP; |  |  |

P_POLY.2 – (BMP) Polygon Perimeter

| Database Name | POLY_PERI |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | Mandatory |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in feet of perimeter of the polygon) |  |  |
| Description | Value in feet of the perimeter of the polygon representing the BMP; Can be calculated within GIS; |  |  |

P_POLY.3 – (BMP) Polygon Surface Area

| Database Name | POLY_SAREA |  |  |
| --- | --- | --- | --- |
| Data Type | Double | Inclusion | Mandatory |
| Width | Default | Domain | (not applicable) |
| Example | (insert value in feet of perimeter of the polygon) |  |  |
| Description | Area value (in acres) of the polygon representing the BMP; Can be calculated within GIS; |  |  |

Treatment of Polygons in the Draft Standard

As is evident throughout this document, preference is given to point geometry as the means to represent non-linear stormwater features. While polygons are important for mapping and visualization, points are to be retained as the main geometry feature for the following reasons:

- The ever-changing shape of shorelines due to seasonal fluctuations, rainfall events, etc.;

- Point features (linked by lines) are better able to preserve the network connectivity of features;

- Points provide a single, continuous place to maintain the associated attribute data;

While the point version of Basins and BMPs are the primary means of representing these features, the point data can easily be spatially joined in GIS software to the polygon features as they are created and needed.

In the example at right, the two constructed basins would primarily be represented in this draft standard as the green points (Basin points at their center). These points would have all the attributes of Basins (from pp. 43-55 of this document) associated with them. The points facilitate connections to the incoming pipes, inlets, outlets via artificial paths.

The polygons (shown in light blue) outlining these basin features would have only the three unique attributes associated with them (as listed on page 159 if a Basin, or page 160 if a BMP) which indicate the method of creation for the polygon, its calculated perimeter and its calculated surface area.

In the lower illustration, the oblique view simply shows that all the data embedded in the point can be quickly spatially joined to the polygon features as needed. Only the data in the point would need to be maintained and updated. A

As new polygons are created being digitized aerial images, extracted from LIDAR, imported from CAD drawing as-builts, etc.

a new spatial join can be performed to update the polygon features attributes.